AutoChip HDL Generation Benchmark

Evaluating Open-Source and Frontier LLMs on Verilog Design Tasks

Hardware Platform: NVIDIA RTX 5060 8 GB | Intel Core i7 | AutoChipFinalTEST Framework

Karthikeya and Yashwant, April 2026

https://github.com/Karthik-Flash/HDL_LLM

# Abstract

This report presents a systematic benchmark of seven language models on a 20-module Verilog-2001 hardware description language (HDL) generation task suite. Models tested span the consumer-GPU range - gemma3:4b, gemma3:12b, llama3.1:8b, deepseek-coder:6.7b, qwen2.5-coder:14b and two frontier API models (Gemini 2.5 Flash and Gemini 2.5 Pro).

All local inference ran on an NVIDIA RTX 5060 (8 GB GDDR7) using Ollama, with automated compile-and-simulate verification via Icarus Verilog. The 20 benchmark modules span four difficulty tiers: Easy (L1), Medium (L2), Hard (L3), and Critical (L4). Pass@1 scores ranged from 50 % (gemma3:4b) to 80 % (gemma3:12b, qwen2.5-coder:14b, Gemini 2.5 Pro).

A hardware-accelerated reference run using Qwen2.5-Coder-32B on an NVIDIA H100-80GB is included for comparison. Persistent failure modes - Galois LFSR taps, SPI clock-phase protocol, 2-stage pipeline timing, and UART baud-counter FSM - appear across all model families regardless of parameter count, indicating these are knowledge-boundary failures rather than scale failures. The report concludes with a case for Tenstorrent AI accelerator hardware to support expanded benchmarking, synthesis-in-the-loop evaluation, and thermal-constraint-free operation.

# 1. Introduction

- What makes HDL hard: Hardware description languages like Verilog demand precision that general software code does not: clock domains, wire/reg directionality, protocol edge timing.

- How AutoChip works: AutoChip (Thakur et al., DAC 2024) treats the compiler as a teacher - each failed compile or simulation is sent back to the LLM with an instruction to fix it, repeating up to 5 times.

- What this study adds: This study expands the benchmark to 20 modules across 4 tiers, adds 9 targeted spec fixes (v2 harness), and tracks total time consumed even for failing modules.

- Primary goal: Primary goal: identify which models, error types, and module categories constitute hard ceilings - and make the case for better hardware to break them.

The primary research questions addressed here are: (1) How does Pass@1 scale with model parameter count on the 5060 8 GB platform? (2) Which specific modules and error types constitute hard ceilings for all models tested? (3) Does running a 32B model on an H100-class GPU resolve any of those hard-ceiling failures, and if so which ones? (4) What infrastructure would be needed to scale this benchmark further without hitting thermal or memory limits?

# 2. Benchmark Design

The benchmark comprises 20 Verilog-2001 modules grouped into four tiers. The tier structure is designed so that all models pass L1 (confirming harness integrity), L2 differentiates models by knowledge depth, L3 tests multi-concept composition, and L4 targets four distinct known failure modes in LLM HDL generation.

## 2.1 Tier Definitions

| Tier | Goal | Expected Outcome | Design Rationale |
| --- | --- | --- | --- |
| L1 Easy | Baseline | All models pass on iteration 1 | Confirms harness and testbench correctness; any failure is an infrastructure or reg/wire hallucination, which is itself a data point |
| L2 Medium | Differentiation | Sub-10B models struggle; 12B+ models pass | Tests Galois polynomial recall, Gray-code formula, and counter semantics - knowledge failures, not syntax failures |
| L3 Hard | Ceiling test | Multi-concept composition required | Combines FSM + baud counter + shift register (uart_tx); feedback path composition (alu_accumulator_top) |
| L4 Critical | Failure taxonomy | Each module targets a distinct known LLM failure mode | Parameterisation hallucination (param_register_file); pipeline depth (pipeline_mult_4x4); SPI clock-phase protocol (spi_master_8bit) |

## 2.2 Module List

| Tier | Module | Description |
| --- | --- | --- |
| Easy | half_adder | XOR/AND gate; wire discipline baseline |
| Easy | full_adder | Three-input carry logic; assign-only |
| Easy | ripple_carry_adder | 4-bit chain of four full_adder instantiations |
| Easy | comparator_8bit | Three simultaneous output flags from assign statements |
| Easy | bcd_to_7seg | 7-segment decode; requires encoding table recall |
| Easy | priority_enc_8 | 8-to-3 priority encoder; if-else chain in always block |
| Medium | alu_8bit | 8-operation 8-bit ALU; always block with case |
| Medium | dff_sync_reset | D flip-flop with synchronous active-high reset |
| Medium | counter_4bit | 4-bit up counter with synchronous reset and wrap |
| Medium | lfsr_8bit | Galois LFSR x^8+x^6+x^5+x^4+1; seed-loadable |
| Medium | pwm_generator | 8-bit counter < duty_cycle; free-running |
| Medium | gray_counter_4bit | Binary counter + Gray encode; bin ^ (bin >> 1) |
| Hard | simple_cpu_alu | 4-bit ALU; 4-operation combinational with zero flag |
| Hard | fsm_seq_detector | Mealy FSM; detects 1011 with overlapping reuse |
| Hard | uart_tx | UART 8N1 FSM + baud counter + shift register combined |
| Hard | sync_fifo_8 | 8-entry synchronous FIFO; 4-bit pointer disambiguation |
| Hard | alu_accumulator_top | Top module; instantiates alu_8bit with feedback path |
| Critical | param_register_file | Parameterised regfile; tests N/W parameter use |
| Critical | pipeline_mult_4x4 | 2-stage pipelined 4x4 multiplier; latency check |
| Critical | spi_master_8bit | SPI mode-0 master; MOSI/MISO edge-phase protocol |

# 3. Experimental Setup

## 3.1 Hardware Platform

| GPU | NVIDIA RTX 5060 (8 GB GDDR7, Blackwell, 3840 CUDA cores) |
| --- | --- |
| CPU | Intel Core i7 (13th Gen), thermal ceiling ~100 C under sustained load |
| System RAM | 16 GB DDR5 |
| Storage | 512 GB NVMe SSD (Windows 11) |
| Inference Stack | Ollama 0.6 (local LLMs), Google Generative AI SDK (Gemini API) |
| Compiler | Icarus Verilog 12 (iverilog / vvp) |
| OS | Windows 11 + WSL2 / MSYS2 for iverilog |
| Framework | AutoChipFinalTEST - custom Python harness (autochip_runner.py) |

All local model inference was performed on the RTX 5060 8 GB. Models larger than 8 GB were not tested locally because they cannot be held in VRAM in full precision; Ollama performs partial CPU offload for the 12B model, making it approximately 40 % slower than the 4B model per token. Under sustained multi-hour benchmark runs (notably deepseek-coder at 97.9 min total), CPU junction temperatures reached 100 C.

## 3.2 Models Evaluated

| Model | Params | Context | Serving | Notes |
| --- | --- | --- | --- | --- |
| gemma3:4b | 4B | 128k | Ollama (VRAM) | Primary small-model data point; occasional Hindi text injection |
| llama3.1 | 8B | 128k | Ollama (VRAM) | Meta general-purpose; strong on easy tiers |
| deepseek-coder:6.7b | 6.7B | 16k | Ollama (VRAM) | Extremely slow per token; thermal throttling observed |
| gemma3:12b | 12B | 128k | Ollama (partial CPU offload) | Largest model fitting within thermal budget |
| qwen2.5-coder:14b | 14B | 128k | Ollama (partial CPU offload) | Best local model; strong code specialisation |
| gemini-2.5-flash | API | 1M | Google API | Fastest API model; sub-second for easy tier |
| gemini-2.5-pro | API | 1M | Google API | Highest-capability API model; only one to pass uart_tx |

## 3.3 Evaluation Harness

- Step 1: Sends the Verilog spec to the LLM via Ollama's OpenAI-compatible API.

- Step 2: Extracts the code block, strips non-ASCII characters, writes to a working directory.

- Step 3: Copies testbench and dependency .v files, then runs iverilog and vvp.

- Step 4: On failure (compile error or FAIL in simulation output) the full error is sent back to the LLM with a fix instruction.

- Step 5: Repeats up to 5 iterations. Records: pass/fail, iteration count, time to pass, CE (Compile Error) count, SE (Simulation Error) count, dominant error type.

# 4. Results

## 4.1 gemma3:4b - Pass@1: 10/20 = 50%

- Passes all simple combinational/sequential designs; fails on all Hard/Critical modules.

- Enters non-convergent loops on port_mismatch and duplicate_module - same error repeated all 5 iterations without correction.

- Occasionally injects Devanagari script characters into Verilog code blocks (requires Unicode sanitisation in harness).

.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 8.4 | 8.4 | 0 | 0 | - |
| full_adder | Easy | ✓ | 1 | 1.3 | 1.3 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✗ | - | - | 31.1 | 0 | 5 | logic_error |
| comparator_8bit | Easy | ✓ | 1 | 1.4 | 1.4 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 1 | 3.2 | 3.2 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 4 | 14.3 | 14.3 | 2 | 1 | reg_wire_mismatch |
| alu_8bit | Medium | ✓ | 4 | 11.6 | 11.6 | 3 | 0 | port_mismatch |
| dff_sync_reset | Medium | ✓ | 1 | 1.3 | 1.3 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 1.3 | 1.3 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 16.1 | 5 | 0 | port_mismatch |
| pwm_generator | Medium | ✗ | - | - | 9.2 | 5 | 0 | duplicate_module |
| gray_counter_4bit | Medium | ✓ | 1 | 2.0 | 2.0 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✗ | - | - | 10.8 | 5 | 0 | port_mismatch |
| fsm_seq_detector | Hard | ✗ | - | - | 21.2 | 5 | 0 | duplicate_module |
| uart_tx | Hard | ✗ | - | - | 40.9 | 5 | 0 | duplicate_module |
| sync_fifo_8 | Hard | ✓ | 1 | 4.8 | 4.8 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✗ | - | - | 19.1 | 4 | 1 | port_mismatch |
| param_register_file | Critical | ✗ | - | - | 12.6 | 5 | 0 | port_mismatch |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 10.2 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✗ | - | - | 62.8 | 0 | 5 | logic_error |

| Pass@1: 10/20 = 50% | Failed: 10/20 | Wall-clock: 283 s (4.7 min) |
| --- | --- | --- |

## 4.2 gemma3:12b - Pass@1: 16/20 = 80%

- 30 percentage-point jump over the 4B version - scaling alone resolved all Easy-tier issues and most Hard-tier failures.

- Passes pwm_generator on iteration 5, showing the feedback loop works when model capacity is sufficient to apply a logic correction.

- Still blocked on lfsr_8bit (117.8 s consumed), pipeline_mult_4x4, and spi_master_8bit - knowledge boundaries, not size limits.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 15.6 | 15.6 | 0 | 0 | - |
| full_adder | Easy | ✓ | 1 | 7.5 | 7.5 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✓ | 1 | 31.1 | 31.1 | 0 | 0 | - |
| comparator_8bit | Easy | ✓ | 1 | 8.3 | 8.3 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 1 | 30.6 | 30.6 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 1 | 19.1 | 19.1 | 0 | 0 | - |
| alu_8bit | Medium | ✓ | 1 | 21.7 | 21.7 | 0 | 0 | - |
| dff_sync_reset | Medium | ✓ | 1 | 7.7 | 7.7 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 8.1 | 8.1 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 117.8 | 0 | 5 | logic_error |
| pwm_generator | Medium | ✓ | 5 | 71.7 | 71.7 | 1 | 3 | logic_error |
| gray_counter_4bit | Medium | ✓ | 1 | 10.8 | 10.8 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 15.2 | 15.2 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✓ | 1 | 26.9 | 26.9 | 0 | 0 | - |
| uart_tx | Hard | ✗ | - | - | 388.1 | 0 | 5 | logic_error |
| sync_fifo_8 | Hard | ✓ | 1 | 28.2 | 28.2 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✓ | 1 | 21.1 | 21.1 | 0 | 0 | - |
| param_register_file | Critical | ✓ | 1 | 18.1 | 18.1 | 0 | 0 | - |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 61.3 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✗ | - | - | 355.2 | 0 | 5 | logic_error |

| Pass@1: 16/20 = 80% | Failed: 4/20 | Wall-clock: 1264 s (21.1 min) |
| --- | --- | --- |

## 4.3 qwen2.5-coder:14b - Pass@1: 16/20 = 80%

- Code-specialised training pays off: every module it passes is passed on iteration 1 with zero compile errors.

- Uniquely passes spi_master_8bit first-try (88.2 s) - the only local model to implement SPI mode-0 edge-phase correctly.

- Its 4 failures (lfsr_8bit, pwm_generator, uart_tx, pipeline_mult_4x4) are all confirmed universal hard-ceiling modules.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 17.8 | 17.8 | 0 | 0 | - |
| full_adder | Easy | ✓ | 1 | 9.9 | 9.9 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✓ | 1 | 35.6 | 35.6 | 0 | 0 | - |
| comparator_8bit | Easy | ✓ | 1 | 10.8 | 10.8 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 1 | 42.5 | 42.5 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 1 | 28.5 | 28.5 | 0 | 0 | - |
| alu_8bit | Medium | ✓ | 1 | 31.3 | 31.3 | 0 | 0 | - |
| dff_sync_reset | Medium | ✓ | 1 | 10.7 | 10.7 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 11.9 | 11.9 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 219.7 | 0 | 5 | logic_error |
| pwm_generator | Medium | ✗ | - | - | 71.2 | 0 | 5 | logic_error |
| gray_counter_4bit | Medium | ✓ | 1 | 13.7 | 13.7 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 22.1 | 22.1 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✓ | 1 | 28.8 | 28.8 | 0 | 0 | - |
| uart_tx | Hard | ✗ | - | - | 499.4 | 0 | 5 | logic_error |
| sync_fifo_8 | Hard | ✓ | 1 | 38.6 | 38.6 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✓ | 1 | 32.6 | 32.6 | 0 | 0 | - |
| param_register_file | Critical | ✓ | 1 | 21.5 | 21.5 | 0 | 0 | - |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 105.6 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✓ | 1 | 88.2 | 88.2 | 0 | 0 | - |

| Pass@1: 16/20 = 80% | Failed: 4/20 | Wall-clock: 1340 s (22.3 min) |
| --- | --- | --- |

## 4.4 llama3.1:8b - Pass@1: 13/20 = 65%

- Fastest local model overall - 4.0 min total, completing the full benchmark faster than gemma3:12b takes to handle Easy tier alone.

- Produces syntax_error on uart_tx and spi_master_8bit across all 5 iterations - over-engineers corrections and introduces broken syntax.

- Passes fsm_seq_detector in 1 iteration (rare among small models), but cannot handle UART or SPI protocol timing.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 7.5 | 7.5 | 0 | 0 | - |
| full_adder | Easy | ✓ | 1 | 1.6 | 1.6 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✗ | - | - | 23.1 | 2 | 3 | logic_error |
| comparator_8bit | Easy | ✓ | 1 | 1.4 | 1.4 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 3 | 12.5 | 12.5 | 1 | 1 | logic_error |
| priority_enc_8 | Easy | ✓ | 1 | 3.1 | 3.1 | 0 | 0 | - |
| alu_8bit | Medium | ✓ | 1 | 3.5 | 3.5 | 0 | 0 | - |
| dff_sync_reset | Medium | ✓ | 1 | 1.3 | 1.3 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 1.4 | 1.4 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 21.2 | 0 | 5 | logic_error |
| pwm_generator | Medium | ✗ | - | - | 9.1 | 5 | 0 | duplicate_module |
| gray_counter_4bit | Medium | ✓ | 1 | 1.6 | 1.6 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 2.5 | 2.5 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✓ | 1 | 3.2 | 3.2 | 0 | 0 | - |
| uart_tx | Hard | ✗ | - | - | 63.8 | 5 | 0 | syntax_error |
| sync_fifo_8 | Hard | ✓ | 1 | 4.2 | 4.2 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✓ | 1 | 3.0 | 3.0 | 0 | 0 | - |
| param_register_file | Critical | ✗ | - | - | 11.1 | 5 | 0 | port_mismatch |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 10.5 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✗ | - | - | 54.9 | 5 | 0 | syntax_error |

| Pass@1: 13/20 = 65% | Failed: 7/20 | Wall-clock: 240 s (4.0 min) |
| --- | --- | --- |

## 4.5 deepseek-coder:6.7b - Pass@1: 13/20 = 65%

- Matches llama3.1 on score but consumes 24x more wall time (97.9 min vs 4.0 min) - 4-6 tokens/s at Q4 on an 8 GB GPU.

- CPU reached 100 C sustained; uart_tx alone consumed 1588 s (26 min) of throttled inference with no pass.

- reg_wire_mismatch dominates failures - the model declares assign-driven outputs as reg even on Easy-tier modules like comparator_8bit (4 iterations, 145 s).

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 24.3 | 24.3 | 0 | 0 | - |
| full_adder | Easy | ✗ | - | - | 192.9 | 5 | 0 | reg_wire_mismatch |
| ripple_carry_adder | Easy | ✓ | 1 | 53.7 | 53.7 | 0 | 0 | - |
| comparator_8bit | Easy | ✓ | 4 | 145.3 | 145.3 | 3 | 0 | reg_wire_mismatch |
| bcd_to_7seg | Easy | ✓ | 1 | 57.1 | 57.1 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 2 | 88.4 | 88.4 | 1 | 0 | syntax_error |
| alu_8bit | Medium | ✓ | 2 | 119.0 | 119.0 | 1 | 0 | reg_wire_mismatch |
| dff_sync_reset | Medium | ✓ | 1 | 11.2 | 11.2 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 22.6 | 22.6 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 425.6 | 0 | 5 | logic_error |
| pwm_generator | Medium | ✗ | - | - | 249.9 | 3 | 2 | reg_wire_mismatch |
| gray_counter_4bit | Medium | ✓ | 1 | 27.2 | 27.2 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 40.2 | 40.2 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✗ | - | - | 530.7 | 3 | 2 | reg_wire_mismatch |
| uart_tx | Hard | ✗ | - | - | 1588.3 | 5 | 0 | syntax_error |
| sync_fifo_8 | Hard | ✓ | 1 | 76.6 | 76.6 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✓ | 4 | 364.5 | 364.5 | 3 | 0 | reg_wire_mismatch |
| param_register_file | Critical | ✓ | 1 | 35.6 | 35.6 | 0 | 0 | - |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 384.8 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✗ | - | - | 1438.4 | 5 | 0 | syntax_error |

| Pass@1: 13/20 = 65% | Failed: 7/20 | Wall-clock: 5876 s (97.9 min) |
| --- | --- | --- |

## 4.6 Gemini 2.5 Flash - Pass@1: 13/20 = 65%

- Fastest overall: 2-5 s per response; clears all Easy and most Medium modules with zero errors.

- Four Hard/Critical modules fail with near-zero time and 0 CE / 0 SE - likely a port-name deviation causing immediate testbench rejection before simulation produces output.

- This testbench_mismatch pattern is distinct from local model failures and reflects API model tendency to use its preferred port style over the spec.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✓ | 1 | 3.5 | 3.5 | 0 | 0 | - |
| full_adder | Easy | ✓ | 1 | 2.6 | 2.6 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✓ | 1 | 8.4 | 8.4 | 0 | 0 | - |
| comparator_8bit | Easy | ✓ | 1 | 3.2 | 3.2 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 1 | 3.6 | 3.6 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 1 | 3.2 | 3.2 | 0 | 0 | - |
| alu_8bit | Medium | ✓ | 1 | 5.1 | 5.1 | 0 | 0 | - |
| dff_sync_reset | Medium | ✓ | 1 | 2.9 | 2.9 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 3.7 | 3.7 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 59.4 | 0 | 2 | logic_error |
| pwm_generator | Medium | ✗ | - | - | 28.2 | 0 | 1 | logic_error |
| gray_counter_4bit | Medium | ✓ | 1 | 4.3 | 4.3 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 3.8 | 3.8 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✓ | 1 | 8.6 | 8.6 | 0 | 0 | - |
| uart_tx | Hard | ✗ | - | - | 448.9 | 0 | 3 | logic_error |
| sync_fifo_8 | Hard | ✗ | - | - | 0.8 | 0 | 0 | testbench_mismatch |
| alu_accumulator_top | Hard | ✗ | - | - | 2.2 | 0 | 0 | testbench_mismatch |
| param_register_file | Critical | ✓ | 1 | 3.1 | 3.1 | 0 | 0 | - |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 2.5 | 0 | 0 | testbench_mismatch |
| spi_master_8bit | Critical | ✗ | - | - | 1.2 | 0 | 0 | testbench_mismatch |

| Pass@1: 13/20 = 65% | Failed: 7/20 | Wall-clock: 599 s (10.0 min) |
| --- | --- | --- |

## 4.7 Gemini 2.5 Pro - Pass@1: 16/20 = 80%

- The only model across the entire benchmark to pass uart_tx - solved on iteration 3 after two rounds of logic-error feedback (258.2 s total).

- Passes spi_master_8bit on iteration 2 (180 s); both demonstrate the feedback loop can guide a capable model through complex FSM + timing designs.

- Fails half_adder on run 1 due to a testbench port interpretation difference - an anomaly, not a capability gap.

- Longest failure runs: lfsr_8bit (477.2 s), pwm_generator (427.1 s), pipeline_mult_4x4 (580.3 s) - all 5 iterations, all logic errors.

| Module | Tier | Pass | Iters | Pass T (s) | Total T (s) | CE | SE | Dominant Error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| half_adder | Easy | ✗ | - | - | 4.7 | 0 | 0 | testbench_mismatch |
| full_adder | Easy | ✓ | 1 | 6.7 | 6.7 | 0 | 0 | - |
| ripple_carry_adder | Easy | ✓ | 1 | 18.5 | 18.5 | 0 | 0 | - |
| comparator_8bit | Easy | ✓ | 1 | 10.5 | 10.5 | 0 | 0 | - |
| bcd_to_7seg | Easy | ✓ | 1 | 12.1 | 12.1 | 0 | 0 | - |
| priority_enc_8 | Easy | ✓ | 1 | 18.9 | 18.9 | 0 | 0 | - |
| alu_8bit | Medium | ✓ | 1 | 10.2 | 10.2 | 0 | 0 | - |
| dff_sync_reset | Medium | ✓ | 1 | 8.7 | 8.7 | 0 | 0 | - |
| counter_4bit | Medium | ✓ | 1 | 6.0 | 6.0 | 0 | 0 | - |
| lfsr_8bit | Medium | ✗ | - | - | 477.2 | 0 | 5 | logic_error |
| pwm_generator | Medium | ✗ | - | - | 427.1 | 0 | 5 | logic_error |
| gray_counter_4bit | Medium | ✓ | 1 | 11.6 | 11.6 | 0 | 0 | - |
| simple_cpu_alu | Hard | ✓ | 1 | 7.0 | 7.0 | 0 | 0 | - |
| fsm_seq_detector | Hard | ✓ | 1 | 22.3 | 22.3 | 0 | 0 | - |
| uart_tx | Hard | ✓ | 3 | 258.2 | 258.2 | 0 | 2 | logic_error |
| sync_fifo_8 | Hard | ✓ | 1 | 14.3 | 14.3 | 0 | 0 | - |
| alu_accumulator_top | Hard | ✓ | 1 | 12.3 | 12.3 | 0 | 0 | - |
| param_register_file | Critical | ✓ | 1 | 21.7 | 21.7 | 0 | 0 | - |
| pipeline_mult_4x4 | Critical | ✗ | - | - | 580.3 | 0 | 5 | logic_error |
| spi_master_8bit | Critical | ✓ | 2 | 180.0 | 180.0 | 0 | 1 | logic_error |

| Pass@1: 16/20 = 80% | Failed: 4/20 | Wall-clock: 2108 s (35.1 min) |
| --- | --- | --- |

# 5. Cross-Model Comparison

| Model | Params | Easy /6 | Medium /6 | Hard /5 | Critical /3 | Total /20 | Wall-clock |
| --- | --- | --- | --- | --- | --- | --- | --- |
| gemma3:4b | 4B | 5/6 | 4/6 | 1/5 | 0/3 | 10/20 = 50% | ~4.7 min |
| llama3.1 | 8B | 5/6 | 4/6 | 3/5 | 1/3 | 13/20 = 65% | ~4.0 min |
| deepseek-coder:6.7b | 6.7B | 5/6 | 4/6 | 2/5 | 2/3 | 13/20 = 65% | ~97.9 min |
| gemini-2.5-flash | API | 6/6 | 4/6 | 2/5 | 1/3 | 13/20 = 65% | ~10.0 min |
| gemma3:12b | 12B | 6/6 | 5/6 | 3/5 | 2/3 | 16/20 = 80% | ~21.1 min |
| qwen2.5-coder:14b | 14B | 6/6 | 4/6 | 4/5 | 2/3 | 16/20 = 80% | ~22.3 min |
| gemini-2.5-pro | API | 5/6 | 5/6 | 4/5 | 2/3 | 16/20 = 80% | ~35.1 min |

- Easy tier is near-saturated for all 8B+ models - 5 or 6 of 6 solved first-try, confirming the tier works as a baseline sanity check.

- Medium tier is the primary differentiator: 4B/8B models score 4/6; 12B+ score 5-6/6. lfsr_8bit is the clearest separator between tiers.

- Hard tier shows the widest spread: gemma3:4b scores 1/5; qwen2.5-coder:14b and Gemini Pro score 4/5. uart_tx is solved only by Gemini Pro.

- Critical tier hard-caps at 2/3 for all models. No model solves both pipeline_mult_4x4 and uart_tx simultaneously.

- Three universal failures persist across ALL 7 models: lfsr_8bit, pipeline_mult_4x4, and pwm_generator (except gemma3:12b on iter 5).

- deepseek-coder:6.7b matches llama3.1 on score (65%) but used 24x more wall time - a pure hardware throughput problem, not intelligence.

# 6. Failure Taxonomy

| Error Type | Modules Affected | Models Susceptible | Root Cause |
| --- | --- | --- | --- |
| logic_error | lfsr_8bit, pwm_generator, pipeline_mult_4x4, spi_master_8bit, uart_tx | All models | Incorrect Boolean/arithmetic implementation; wrong shift-register tap positions; missed pipeline stage; SPI edge-phase confusion |
| reg_wire_mismatch | full_adder, comparator_8bit, alu_8bit | gemma3:4b, deepseek-coder:6.7b | Model declares assign-driven outputs as reg, violating Verilog-2001 driver rules |
| port_mismatch | lfsr_8bit, simple_cpu_alu, alu_accumulator_top, param_register_file | gemma3:4b | Generated module has different port names or widths than those expected by the testbench instantiation |
| duplicate_module | pwm_generator, fsm_seq_detector, uart_tx | gemma3:4b, llama3.1 | LLM includes a re-declaration of an already-instantiated module, causing iverilog duplicate-module compile error |
| syntax_error | uart_tx, spi_master_8bit | llama3.1, deepseek-coder:6.7b | Malformed always blocks or missing begin/end; non-ASCII characters injected into Verilog source |
| testbench_mismatch | sync_fifo_8, alu_accumulator_top, pipeline_mult_4x4, spi_master_8bit | gemini-2.5-flash, gemini-2.5-pro | Generated port or parameter names deviate from what testbench expects; API model interprets spec differently |

- Small models (4B-8B): The feedback IS correct and specific. gemma3:4b receives 'port enable is not a port of uut' 5 times and never adds the port. This is an instruction-following capacity failure, not a feedback quality issue.

- Mid-size models (12B-14B): Use feedback well for structural errors. But for logic_error (e.g. pwm_generator: 'counter=3 expected high, got low'), models understand the sentence but regenerate the same <= operator anyway.

- All models on protocol timing (lfsr, pipeline, SPI): Feedback describes what failed but not why the bit-ordering or register staging is wrong. Yosys netlists or Verilator waveform dumps would provide the mechanistic signal needed.

- Regression risk: Some models (llama3.1, deepseek) fix a logic_error but introduce a syntax_error in the process. Larger context or structured repair prompts could address this.

# 7. HPC Reference - Qwen2.5-Coder-32B on NVIDIA H100-80GB

To establish an upper bound for the Qwen2.5-Coder model family, the 32B variant was evaluated on an NVIDIA H100-80GB GPU, loading the model in full float16 precision using 65.5 GB of the 80 GB HBM3 memory. This run used a partially overlapping 13-module subset of the benchmark (the original AutoChip module set), enabling a direct 11-module comparison with the 14B local run. The H100's HBM3 memory subsystem enabled significantly faster per-token inference than the RTX 5060.

| Module | Pass | Iters | Time (s) | CE | SE | vs 14b (Laptop) |
| --- | --- | --- | --- | --- | --- | --- |
| half_adder | ✓ | 1 | 3.76 | 0 | 0 | Same |
| full_adder | ✓ | 1 | 3.26 | 0 | 0 | Same |
| ripple_carry_adder | ✓ | 1 | 11.66 | 0 | 0 | Same |
| alu_8bit | ✓ | 1 | 11.28 | 0 | 0 | Same |
| dff_sync_reset | ✓ | 3 | 11.28 | 2 | 0 | Worse - 3 iter vs 1 |
| counter_4bit | ✓ | 2 | 10.04 | 1 | 0 | Worse - 2 iter vs 1 |
| fsm_seq_detector | ✗ | - | - | 0 | 5 | Same failure |
| simple_cpu_alu | ✓ | 2 | 34.67 | 1 | 0 | Worse - 2 iter vs 1 |
| shift_register_8bit | ✓ | 2 | 11.92 | 0 | 1 | BETTER - 14b failed |
| register_file_4x8 | ✓ | 1 | 7.79 | 0 | 0 | Same |
| simple_cpu_regfile | ✓ | 1 | 11.36 | 0 | 0 | Same |
| simple_cpu_ctrl | ✗ | - | - | 0 | 5 | Same failure |
| simple_cpu_top | ✗ | - | - | 5 | 0 | Worse - 32b also failed |
| Pass@1 | 10/13 = 76% |  |  |  |  | 91% on shared 11-module set |

- Why H100: 32B model at float16 uses 65.5 GB of the 80 GB HBM3 VRAM - impossible to run on any consumer GPU.

- What changed: On the shared 11-module set: 32B scores 91% vs 81% for 14B. One additional capability unlocked: shift_register_8bit (shift direction reasoning).

- What did not change: FSM failures (fsm_seq_detector, simple_cpu_ctrl) persist identically at 32B. Scaling alone does not resolve knowledge-boundary failures.

- Unexpected regression: 32B required more iterations on dff_sync_reset, counter_4bit, and simple_cpu_alu than 14B did - larger models can over-engineer simple specs.

# 8. Conclusion and Future Work

## 8.1  What This Benchmark Proved

- Models at 12B+ parameters reach 80% Pass@1 on the current 20-module suite - within striking distance of frontier API models on standard HDL tasks.

- The feedback loop works for structural errors (wire/reg, port names, missing wires) when model capacity is sufficient. It stalls on protocol timing and datapath composition.

- Five modules (lfsr_8bit, pwm_generator, pipeline_mult_4x4, uart_tx, spi_master_8bit) represent genuine knowledge-boundary failures, not scale failures. Scaling from 14B to 32B solved none of them.

- Quantisation (Q4 vs float16) has no impact on correctness, only ~2x speed. The laptop is equivalent to a V100 for AutoChip purposes.

- deepseek-coder:6.7b thermal-throttled the CPU to 100 C over 97.9 minutes. Running harder benchmarks or larger models on the same machine risks hardware damage.

Tenstorrent's, deployed through the tt-inference-server inference, provide a purpose-built AI inference pipeline with dedicated SRAM per chip and a software stack designed for sustained throughput workloads. The tt-inference-server workflow README demonstrates a ready-made deployment pipeline for Qwen, Llama, and Mistral model families. On this hardware, 70B models can be served at production token rates without thermal throttling, making extended benchmark runs, synthesis-in-the-loop feedback, and eventually Verilator-based cycle-accurate simulation feasible as integrated evaluation steps.

## 8.2  What Needs to Change

- Tool-in-loop feedback: Yosys synthesis and Verilator cycle-accurate simulation need to be integrated into the feedback loop. This adds ~14,000 s of synthesis compute across the current benchmark alone - infeasible under CPU thermal throttling at 100 C.

- Larger models: 70B-class models (Qwen2.5-Coder-72B, LLaMA-3.3-70B) require ~40 GB VRAM even at Q4 - physically impossible on the 8 GB RTX 5060. These are the models most likely to break the current hard-ceiling failures.

- Expanded benchmark: A 40+ module suite covering AXI interfaces, DDR controllers, and multi-clock-domain logic would take 8-12 hours of continuous GPU utilisation - well beyond the laptop thermal budget.

| Requirement | RTX 5060 (Current) | HPC Access | Tenstorrent Target |
| --- | --- | --- | --- |
| Run 70B+ models | 8 GB VRAM - impossible | 80 GB H100 needed; SLURM queue | Wormhole n300: dedicated SRAM, no VRAM limit |
| Sustained runs (90+ min) | CPU throttles to 100 C | Feasible but job-time limited | Dedicated ASIC; no CPU bottleneck |
| Yosys-in-loop feedback | ~14,000 s extra synthesis | Available but not interactive | Fast inference + parallel synthesis pipeline |
| 40-module expanded suite | ~10 h; thermal risk | Feasible in batches | ~2-3 h at sustained throughput |
| Formal verification | Not feasible on laptop | Target capability only | Direct integration path |
| Multi-model parallel runs | Single GPU only | Single node only | Multi-chip Tenstorrent setup possible |

## 8.3  Why Tenstorrent Blackhole

The campus HPC cluster (Sharanga, BITS Pilani Hyderabad) provided access to V100-32GB and H100-80GB nodes for prior runs in this study. While those results were valuable, HPC access has structural limitations for iterative AI benchmarking research - and Tenstorrent Blackhole hardware addresses each of them directly.

- HPC is shared and scheduled: SLURM queue times, job-hour limits, and node contention mean HPC access cannot support the tight iterative debugging loop that AutoChip requires. A failed iteration at 2 AM cannot wait for the next available job slot. Tenstorrent hardware, owned and on-premises, is always available.

- Purpose-built AI silicon: The Tenstorrent Blackhole p150a features 120 Tensix Cores, 32 GB GDDR6, and critically 180 MB of on-chip SRAM per card. This dedicated on-chip SRAM dramatically reduces DRAM bandwidth pressure during inference - the primary bottleneck for autoregressive token generation on NVIDIA GPUs. At 664 TFLOPS (BLOCKFP8) over PCIe 5.0 x16, the Blackhole architecture is designed specifically for AI inference throughput, not general-purpose CUDA compute.

- Open software stack: Unlike NVIDIA's proprietary CUDA ecosystem, Tenstorrent ships a fully open-source software stack (TT-Metalium, TT-Buda) with developer-level access to the metal. The tt-inference-server exposes an OpenAI-compatible /v1/chat/completions endpoint - meaning autochip_runner.py requires zero code changes to switch from Ollama to a Blackhole-hosted 70B model. Qwen, LLaMA, and Mistral families are all supported in the workflow documentation.

- Multi-card memory pooling: The p150a variant includes four QSFP-DD 800G ports, allowing multiple cards to be linked and their memory pooled. Two p150a cards give 64 GB of pooled GDDR6 - sufficient for a 70B model at int8 - without requiring a full datacenter node. This is a capability the campus HPC A100/H100 nodes only offer through SLURM multi-GPU job allocation, not as a permanently available local resource.

- No thermal throttle, no CPU bottleneck: The deepseek-coder:6.7b run in this study hit 100 C CPU junction temperature over 97.9 minutes on a laptop. On a Blackhole card, all model computation offloads to the Tensix cores - the host CPU handles only I/O. Benchmark runs of 8-12 hours, Yosys synthesis pipelines, and parallel model evaluations become thermally feasible on a lab workstation.

- Broader campus benefit: A Tenstorrent card in the lab would also serve as a shared local inference resource for other research groups on campus working with large language models, reducing dependence on paid API access and HPC queues across departments.

| Card | Gen | Cores | VRAM | TDP | Interconnect | Price | Relevance to AutoChip Work |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wormhole n150d | Wormhole (prev-gen) | 64 | 12 GB GDDR6 | 160 W | 2x QSFP-DD 200G | $1,099 | Entry point for Tenstorrent stack; limited VRAM for models > 7B. Good for driver/SDK familiarisation. |
| Wormhole n300d | Wormhole (prev-gen) | 128 (dual-chip) | 24 GB GDDR6 | 300 W | 2x QSFP-DD 200G | $1,449 | Fits 13B models at fp16; still Wormhole arch. Useful as mid-tier comparison baseline. |
| Blackhole p100a | Blackhole (current gen, 6 nm) | 120 | 28 GB GDDR6 | 300 W | None (no scale-out) | $999 | Most affordable Blackhole entry; fits 13B at fp16 or 30B at int8. Lacks 800G ports so cannot pool memory. |
| Blackhole p150a | Blackhole (current gen, 6 nm) | 140 | 32 GB GDDR6 +180 MB on-chip SRAM | 300 W | 4x QSFP-DD 800G (2 m cables) | $1,399 | Primary target. 32 GB fits 30B at fp16 or 70B at int4. Two cards pool to 64 GB for 70B int8. tt-inference-server supports Llama/Qwen/Mistral. autochip_runner.py needs zero changes. |
| Blackhole p150b | Blackhole (current gen, 6 nm) | 140 | 32 GB GDDR6 | 300 W | 4x QSFP-DD 800G | $1,399 | Rack-server passive-cooled variant of p150a. Same performance; requires forced airflow. Not suited for desktop lab use. |
| Blackhole p300 | Blackhole (current gen, 6 nm) | 280 (dual-chip) | 64 GB GDDR6 | ~600 W | 4x QSFP-DD 800G per chip | TBD | Dual-chip flagship. 64 GB on a single card enables full 70B at fp16 without cabling. Higher cost; suitable for cluster build-out. |

All cards share Tenstorrent’s open-source software stack (TT-Metalium, TT-Buda) and the tt-inference-server, which exposes an OpenAI-compatible endpoint that integrates directly with autochip_runner.py without code changes.