# Version 2

## All 8 Fixes Applied

FIX-1 — classify_error now catches reg_wire_mismatch correctly The old code looked for "driven by" or "continuous" — strings iverilog never actually emits. Now it correctly catches "not a valid l-value". This affected uart_tx (12b) where tx_out was a wire being driven in always. The LLM kept getting a generic "compile_other" hint instead of a targeted reg/wire correction hint.

FIX-2 — File paths stripped from error messages before sending to LLM The old code sent results\gemma3_12b\ripple_carry_adder\iter_3\ripple_carry_adder.v:56: verbatim. Now it's cleaned to just ripple_carry_adder.v:56:. Models spend context window trying to interpret paths they can't act on.

FIX-3 — Type-specific hint messages per error category Every error type now gets a concrete repair instruction: reg_wire_mismatch tells the LLM exactly to change wire → reg, duplicate_module tells it to remove its inline copy, port_mismatch tells it not to rename ports, etc.

FIX-4 — ripple_carry_adder spec: Added "Do NOT define or redeclare the full_adder module in this file". Both gemma3 models failed this identically — they inline a copy, which conflicts with the provided full_adder.v.

FIX-5 — uart_tx spec: Replaced $clog2(BAUD_DIV) with reg [7:0] baud_cnt. Older iverilog builds don't support $clog2() in port-width context — this was a 5/5 repeating compile failure across all models.

FIX-6 — pwm_generator spec: Added explicit example: "HIGH for counter=0,1,2,3 when duty=4, LOW for 4..255". All 3 models used <= instead of < — the spec was ambiguous about the boundary.

FIX-7 — lfsr_8bit spec: Named the exact tap positions in plain text with inline comments on each concatenation bit. The polynomial notation alone wasn't enough for smaller models.

FIX-8 — pipeline_mult_4x4 spec: Added "Inputs at cycle N → output at cycle N+2. Testbench checks at exactly N+2." All models implemented it as 1-stage and got got 12 (previous input's result) — a classic pipeline-depth confusion.

# GEMMA 4B

resultsalu_8bit.v:17: error: Unable to bind wire/reg/memory `B' in `alu_8bit_tb.uut'

resultsalu_8bit.v:20: error: Unable to bind wire/reg/memory `B' in `alu_8bit_tb.uut'

resultsalu_8bit.v:23: error: Unable to bind wire/reg/memory `B' in `alu_8bit_tb.uut'

resultsalu_8bit.v:8: error: Unable to bind wire/reg/memory `B' in `alu_8bit_tb.uut'

B

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 5.9s

⚙️  iverilog:

resultsalu_8bit_tb.v:56: Module alu_8bit_tb was already declared here: resultsalu_8bit.v:44

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 5.9s

⚙️  iverilog:

resultsalu_8bit.v:58: error: Net ``zero'' has already been declared.

resultsalu_8bit_tb.v:56: Module alu_8bit_tb was already declared here: resultsalu_8bit.v:44

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 5.9s

⚙️  iverilog:

resultsalu_8bit.v:58: error: Net ``zero'' has already been declared.

resultsalu_8bit_tb.v:56: Module alu_8bit_tb was already declared here: resultsalu_8bit.v:44

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 5.9s

⚙️  iverilog:

resultsalu_8bit.v:58: error: Net ``zero'' has already been declared.

resultsalu_8bit_tb.v:56: Module alu_8bit_tb was already declared here: resultsalu_8bit.v:44

❌ duplicate_module  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: dff_sync_reset  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.3s

✅ PASSED on iteration 1  (1.4s total)

============================================================

AutoChip: counter_4bit  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.2s

✅ PASSED on iteration 1  (1.3s total)

============================================================

AutoChip: lfsr_8bit  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.0s

⚙️  iverilog:

resultslfsr_8bit_tb.v:13: error: port ``enable'' is not a port of uut.

resultslfsr_8bit.v:14: error: Unable to bind wire/reg/memory `enable' in `lfsr_8bit_tb.uut'

resultslfsr_8bit.v:14: error: Unable to elaborate condition expression.

3 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 3.0s

⚙️  iverilog:

resultslfsr_8bit_tb.v:13: error: port ``enable'' is not a port of uut.

resultslfsr_8bit.v:14: error: Unable to bind wire/reg/memory `enable' in `lfsr_8bit_tb.uut'

resultslfsr_8bit.v:14: error: Unable to elaborate condition expression.

3 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 3.1s

⚙️  iverilog:

resultslfsr_8bit_tb.v:13: error: port ``enable'' is not a port of uut.

resultslfsr_8bit.v:14: error: Unable to bind wire/reg/memory `enable' in `lfsr_8bit_tb.uut'

resultslfsr_8bit.v:14: error: Unable to elaborate condition expression.

3 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 3.1s

⚙️  iverilog:

resultslfsr_8bit_tb.v:13: error: port ``enable'' is not a port of uut.

resultslfsr_8bit.v:14: error: Unable to bind wire/reg/memory `enable' in `lfsr_8bit_tb.uut'

resultslfsr_8bit.v:14: error: Unable to elaborate condition expression.

3 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 3.1s

⚙️  iverilog:

resultslfsr_8bit_tb.v:13: error: port ``enable'' is not a port of uut.

resultslfsr_8bit.v:14: error: Unable to bind wire/reg/memory `enable' in `lfsr_8bit_tb.uut'

resultslfsr_8bit.v:14: error: Unable to elaborate condition expression.

3 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.9s

✅ PASSED on iteration 1  (2.0s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.2s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:15: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:17: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 2.2s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:15: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:17: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 2.2s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:15: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:17: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 2.3s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:15: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:17: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 2.3s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:15: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:17: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: fsm_seq_detector  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.8s

⚙️  iverilog:

resultsfsm_seq_detector.v:9: error: Net ``detected'' has already been declared.

resultsfsm_seq_detector.v:25: error: unmatched character (')

resultsfsm_seq_detector.v:25: syntax error

resultsfsm_seq_detector.v:25: error: malformed statement

resultsfsm_seq_detector.v:33: error: unmatched character (')

resultsfsm_seq_detector.v:33: syntax error

resultsfsm_seq_detector.v:33: error: malformed statement

resultsfsm_seq_detector.v:37: error: unmatched character (')

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 3.9s

⚙️  iverilog:

resultsfsm_seq_detector.v:9: error: Net ``detected'' has already been declared.

resultsfsm_seq_detector.v:25: error: unmatched character (')

resultsfsm_seq_detector.v:25: syntax error

resultsfsm_seq_detector.v:25: error: malformed statement

resultsfsm_seq_detector.v:33: error: unmatched character (')

resultsfsm_seq_detector.v:33: syntax error

resultsfsm_seq_detector.v:33: error: malformed statement

resultsfsm_seq_detector.v:37: error: unmatched character (')

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 3.8s

⚙️  iverilog:

resultsfsm_seq_detector.v:9: error: Net ``detected'' has already been declared.

resultsfsm_seq_detector.v:24: error: unmatched character (')

resultsfsm_seq_detector.v:24: syntax error

resultsfsm_seq_detector.v:24: error: malformed statement

resultsfsm_seq_detector.v:32: error: unmatched character (')

resultsfsm_seq_detector.v:32: syntax error

resultsfsm_seq_detector.v:32: error: malformed statement

resultsfsm_seq_detector.v:36: error: unmatched character (')

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 3.8s

⚙️  iverilog:

resultsfsm_seq_detector.v:9: error: Net ``detected'' has already been declared.

resultsfsm_seq_detector.v:24: error: unmatched character (')

resultsfsm_seq_detector.v:24: syntax error

resultsfsm_seq_detector.v:24: error: malformed statement

resultsfsm_seq_detector.v:32: error: unmatched character (')

resultsfsm_seq_detector.v:32: syntax error

resultsfsm_seq_detector.v:32: error: malformed statement

resultsfsm_seq_detector.v:36: error: unmatched character (')

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 3.8s

⚙️  iverilog:

resultsfsm_seq_detector.v:9: error: Net ``detected'' has already been declared.

resultsfsm_seq_detector.v:24: error: unmatched character (')

resultsfsm_seq_detector.v:24: syntax error

resultsfsm_seq_detector.v:24: error: malformed statement

resultsfsm_seq_detector.v:32: error: unmatched character (')

resultsfsm_seq_detector.v:32: syntax error

resultsfsm_seq_detector.v:32: error: malformed statement

resultsfsm_seq_detector.v:36: error: unmatched character (')

❌ duplicate_module  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: uart_tx  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.7s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'hfc  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'hfc  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.8s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'hfc  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'hfc  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 7.7s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'hfc  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'hfc  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 7.9s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'hfc  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'hfc  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 9.9s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'hfc  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'hfc  (bits wrong)

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.4s

⚙️  iverilog:

resultssync_fifo_8.v:17: syntax error

resultssync_fifo_8.v:17: error: Invalid module instantiation

resultssync_fifo_8.v:18: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 4.4s

⚙️  iverilog:

resultssync_fifo_8.v:17: syntax error

resultssync_fifo_8.v:17: error: Invalid module instantiation

resultssync_fifo_8.v:18: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 4.4s

⚙️  iverilog:

resultssync_fifo_8.v:17: syntax error

resultssync_fifo_8.v:17: error: Invalid module instantiation

resultssync_fifo_8.v:18: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 4.4s

⚙️  iverilog:

resultssync_fifo_8.v:17: syntax error

resultssync_fifo_8.v:17: error: Invalid module instantiation

resultssync_fifo_8.v:18: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 4.4s

⚙️  iverilog:

resultssync_fifo_8.v:17: syntax error

resultssync_fifo_8.v:17: error: Invalid module instantiation

resultssync_fifo_8.v:18: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

⚠️  Dependency alu_8bit.v not found for alu_accumulator_top (did the dependency module pass?)

============================================================

AutoChip: alu_accumulator_top  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.7s

⚙️  iverilog:

resultsalu_accumulator_top.v:16: syntax error

resultsalu_accumulator_top.v:15: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 3.7s

⚙️  iverilog:

resultsalu_accumulator_top.v:16: syntax error

resultsalu_accumulator_top.v:15: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 3.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:16: syntax error

resultsalu_accumulator_top.v:15: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 3.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:16: syntax error

resultsalu_accumulator_top.v:15: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 3.6s

⚙️  iverilog:

resultsalu_accumulator_top.v:16: syntax error

resultsalu_accumulator_top.v:15: error: Invalid module instantiation

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: param_register_file  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.4s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 2.3s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 2.3s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 2.4s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 2.4s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pipeline_mult_4x4  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 1.9s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 1.8s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: gemma3:4b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 9.2s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 9.3s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 9.2s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 9.1s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 9.0s

⚙️  iverilog:

resultsspi_master_8bit.v:17: syntax error

resultsspi_master_8bit.v:17: error: syntax error in reg variable list.

resultsspi_master_8bit.v:35: syntax error

resultsspi_master_8bit.v:35: Syntax in assignment statement l-value.

resultsspi_master_8bit.v:48: syntax error

resultsspi_master_8bit.v:51: syntax error

resultsspi_master_8bit.v:53: syntax error

resultsspi_master_8bit.v:56: syntax error

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: gemma3:4b

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1     9.36    0    0

full_adder                   easy         ✓      1      1.5    0    0

ripple_carry_adder           easy         ✗      -        -    5    0

comparator_8bit              easy         ✓      1     1.87    0    0

bcd_to_7seg                  easy         ✗      -        -    0    5

priority_enc_8               easy         ✗      -        -    5    0

alu_8bit                     medium       ✗      -        -    5    0

dff_sync_reset               medium       ✓      1     1.39    0    0

counter_4bit                 medium       ✓      1     1.28    0    0

lfsr_8bit                    medium       ✗      -        -    5    0

pwm_generator                medium       ✗      -        -    0    5

gray_counter_4bit            medium       ✓      1     1.99    0    0

simple_cpu_alu               hard         ✗      -        -    5    0

fsm_seq_detector             hard         ✗      -        -    5    0

uart_tx                      hard         ✗      -        -    0    5

sync_fifo_8                  hard         ✗      -        -    5    0

alu_accumulator_top          hard         ✗      -        -    5    0

param_register_file          critic       ✗      -        -    5    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✗      -        -    1    4

Pass@1: 6/20 = 30%

Summary → results\gemma3_4b\summary.json

# GEMMA 12B

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model gemma3:12b

🚀 Running benchmark | Model: gemma3:12b

============================================================

AutoChip: half_adder  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.2s

✅ PASSED on iteration 1  (16.3s total)

============================================================

AutoChip: full_adder  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.9s

✅ PASSED on iteration 1  (8.0s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 39.1s

✅ PASSED on iteration 1  (39.2s total)

============================================================

AutoChip: comparator_8bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.5s

✅ PASSED on iteration 1  (8.6s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 31.8s

✅ PASSED on iteration 1  (31.9s total)

============================================================

AutoChip: priority_enc_8  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 20.4s

✅ PASSED on iteration 1  (20.6s total)

============================================================

AutoChip: alu_8bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 23.0s

✅ PASSED on iteration 1  (23.1s total)

============================================================

AutoChip: dff_sync_reset  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.8s

✅ PASSED on iteration 1  (8.0s total)

============================================================

AutoChip: counter_4bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.6s

✅ PASSED on iteration 1  (8.7s total)

============================================================

AutoChip: lfsr_8bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 25.0s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'hfc)

FAIL [hold2]: value changed when enable=0 (got 8'hfc)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 26.6s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'hc7

FAIL [AB step1]: expected 8'hC9 got 8'h93

FAIL [AB step2]: expected 8'hF8 got 8'hab

FAIL [hold]: value changed when enable=0 (got 8'h93)

FAIL [hold2]: value changed when enable=0 (got 8'h93)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 26.0s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h8e

FAIL [AB step1]: expected 8'hC9 got 8'ha6

FAIL [AB step2]: expected 8'hF8 got 8'hcc

FAIL [hold]: value changed when enable=0 (got 8'h98)

FAIL [hold2]: value changed when enable=0 (got 8'h98)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 25.7s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h8e

FAIL [AB step1]: expected 8'hC9 got 8'ha6

FAIL [AB step2]: expected 8'hF8 got 8'hcc

FAIL [hold]: value changed when enable=0 (got 8'h98)

FAIL [hold2]: value changed when enable=0 (got 8'h98)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 25.7s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h8e

FAIL [AB step1]: expected 8'hC9 got 8'ha6

FAIL [AB step2]: expected 8'hF8 got 8'hcc

FAIL [hold]: value changed when enable=0 (got 8'h98)

FAIL [hold2]: value changed when enable=0 (got 8'h98)

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.7s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 14.0s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 14.2s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 14.4s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 14.2s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.5s

✅ PASSED on iteration 1  (11.6s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 18.6s

✅ PASSED on iteration 1  (18.7s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 28.6s

✅ PASSED on iteration 1  (28.8s total)

============================================================

AutoChip: uart_tx  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 67.7s

⚙️  iverilog:

resultsuart_tx.v:33: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:41: syntax error

resultsuart_tx.v:49: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:63: error: Incomprehensible case expression.

resultsuart_tx.v:65: syntax error

resultsuart_tx.v:72: error: Incomprehensible case expression.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 66.7s

⚙️  iverilog:

resultsuart_tx.v:33: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:41: syntax error

resultsuart_tx.v:49: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:63: error: Incomprehensible case expression.

resultsuart_tx.v:65: syntax error

resultsuart_tx.v:72: error: Incomprehensible case expression.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 67.4s

⚙️  iverilog:

resultsuart_tx.v:33: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:41: syntax error

resultsuart_tx.v:49: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:63: error: Incomprehensible case expression.

resultsuart_tx.v:65: syntax error

resultsuart_tx.v:72: error: Incomprehensible case expression.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 67.8s

⚙️  iverilog:

resultsuart_tx.v:33: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:41: syntax error

resultsuart_tx.v:49: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:63: error: Incomprehensible case expression.

resultsuart_tx.v:65: syntax error

resultsuart_tx.v:72: error: Incomprehensible case expression.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 74.0s

⚙️  iverilog:

resultsuart_tx.v:32: error: tx_out_internal is not a valid l-value in uart_tx_tb.uut.

resultsuart_tx.v:17:      : tx_out_internal is declared here as wire.

resultsuart_tx.v:41: error: tx_out_internal is not a valid l-value in uart_tx_tb.uut.

resultsuart_tx.v:17:      : tx_out_internal is declared here as wire.

resultsuart_tx.v:59: error: tx_out_internal is not a valid l-value in uart_tx_tb.uut.

resultsuart_tx.v:17:      : tx_out_internal is declared here as wire.

resultsuart_tx.v:66: error: tx_out_internal is not a valid l-value in uart_tx_tb.uut.

resultsuart_tx.v:17:      : tx_out_internal is declared here as wire.

❌ reg_wire_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 30.1s

✅ PASSED on iteration 1  (30.2s total)

============================================================

AutoChip: alu_accumulator_top  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 22.4s

✅ PASSED on iteration 1  (22.6s total)

============================================================

AutoChip: param_register_file  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 15.3s

✅ PASSED on iteration 1  (15.5s total)

============================================================

AutoChip: pipeline_mult_4x4  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 12.6s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 12.8s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 13.0s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 13.2s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 13.0s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: gemma3:12b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 71.0s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 71.0s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 71.6s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 71.5s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 71.0s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'h00

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: gemma3:12b

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1    16.35    0    0

full_adder                   easy         ✓      1     8.04    0    0

ripple_carry_adder           easy         ✓      1    39.22    0    0

comparator_8bit              easy         ✓      1     8.64    0    0

bcd_to_7seg                  easy         ✓      1    31.94    0    0

priority_enc_8               easy         ✓      1    20.58    0    0

alu_8bit                     medium       ✓      1    23.15    0    0

dff_sync_reset               medium       ✓      1     7.97    0    0

counter_4bit                 medium       ✓      1     8.69    0    0

lfsr_8bit                    medium       ✗      -        -    0    5

pwm_generator                medium       ✗      -        -    0    5

gray_counter_4bit            medium       ✓      1    11.63    0    0

simple_cpu_alu               hard         ✓      1    18.69    0    0

fsm_seq_detector             hard         ✓      1    28.78    0    0

uart_tx                      hard         ✗      -        -    5    0

sync_fifo_8                  hard         ✓      1     30.2    0    0

alu_accumulator_top          hard         ✓      1    22.56    0    0

param_register_file          critic       ✓      1    15.45    0    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✗      -        -    0    5

Pass@1: 15/20 = 75%

Summary → results\gemma3_12b\summary.json

# LLAMA 3.1

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model llama3.1

🚀 Running benchmark | Model: llama3.1

============================================================

AutoChip: half_adder  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.8s

✅ PASSED on iteration 1  (9.0s total)

============================================================

AutoChip: full_adder  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.5s

✅ PASSED on iteration 1  (2.6s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.9s

⚙️  iverilog:

resultsripple_carry_adder.v:13: warning: input port cin is coerced to inout.

Assertion failed: &l != &r, file ../verilog-0.9.7/net_link.cc, line 61

❌ compile_other  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 9.3s

⚙️  iverilog:

resultsripple_carry_adder.v:13: error: reg Sum; cannot be driven by primitives or continuous assignment.

resultsripple_carry_adder.v:13: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:13:      : Port sum of full_adder is connected to Sum['sd0]

resultsripple_carry_adder.v:14: error: reg Sum; cannot be driven by primitives or continuous assignment.

resultsripple_carry_adder.v:14: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:14:      : Port sum of full_adder is connected to Sum['sd1]

resultsripple_carry_adder.v:15: error: reg Sum; cannot be driven by primitives or continuous assignment.

resultsripple_carry_adder.v:15: error: Output port expression must support continuous assignment.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 8.7s

⚙️  iverilog:

resultsripple_carry_adder.v:13: error: can not select part of scalar: Sum

resultsripple_carry_adder.v:13: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:13:      : Port sum of full_adder is connected to Sum['sd0]

resultsripple_carry_adder.v:14: error: can not select part of scalar: Sum

resultsripple_carry_adder.v:14: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:14:      : Port sum of full_adder is connected to Sum['sd1]

resultsripple_carry_adder.v:15: error: can not select part of scalar: Sum

resultsripple_carry_adder.v:15: error: Output port expression must support continuous assignment.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 8.8s

⚙️  iverilog:

resultsripple_carry_adder.v:16: error: reg cout; cannot be driven by primitives or continuous assignment.

resultsripple_carry_adder.v:16: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:16:      : Port cout of full_adder is connected to cout

2 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 10.3s

⚙️  iverilog:

resultsripple_carry_adder.v:22: error: reg cout; cannot be driven by primitives or continuous assignment.

resultsripple_carry_adder.v:22: error: Output port expression must support continuous assignment.

resultsripple_carry_adder.v:22:      : Port cout of full_adder is connected to cout

resultsripple_carry_adder.v:14: error: fa1_cin is not a valid l-value in ripple_carry_adder_tb.uut.

resultsripple_carry_adder.v:9:      : fa1_cin is declared here as wire.

resultsripple_carry_adder.v:15: error: fa2_cin is not a valid l-value in ripple_carry_adder_tb.uut.

resultsripple_carry_adder.v:10:      : fa2_cin is declared here as wire.

resultsripple_carry_adder.v:16: error: fa3_cin is not a valid l-value in ripple_carry_adder_tb.uut.

❌ reg_wire_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: comparator_8bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.8s

✅ PASSED on iteration 1  (2.9s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.0s

✅ PASSED on iteration 1  (8.1s total)

============================================================

AutoChip: priority_enc_8  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 6.3s

✅ PASSED on iteration 1  (6.3s total)

============================================================

AutoChip: alu_8bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.3s

⚙️  iverilog:

resultsalu_8bit.v:9: syntax error

resultsalu_8bit.v:9: error: Invalid module instantiation

resultsalu_8bit.v:12: error: invalid module item.

resultsalu_8bit.v:13: syntax error

resultsalu_8bit.v:13: error: invalid module item.

resultsalu_8bit.v:14: syntax error

resultsalu_8bit.v:14: error: invalid module item.

resultsalu_8bit.v:15: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.5s

⚙️  iverilog:

resultsalu_8bit.v:9: syntax error

resultsalu_8bit.v:9: error: Invalid module instantiation

resultsalu_8bit.v:12: error: invalid module item.

resultsalu_8bit.v:13: syntax error

resultsalu_8bit.v:13: error: invalid module item.

resultsalu_8bit.v:14: syntax error

resultsalu_8bit.v:14: error: invalid module item.

resultsalu_8bit.v:15: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 7.4s

⚙️  iverilog:

resultsalu_8bit.v:9: syntax error

resultsalu_8bit.v:9: error: Invalid module instantiation

resultsalu_8bit.v:12: error: invalid module item.

resultsalu_8bit.v:13: syntax error

resultsalu_8bit.v:13: error: invalid module item.

resultsalu_8bit.v:14: syntax error

resultsalu_8bit.v:14: error: invalid module item.

resultsalu_8bit.v:15: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 7.4s

⚙️  iverilog:

resultsalu_8bit.v:9: syntax error

resultsalu_8bit.v:9: error: Invalid module instantiation

resultsalu_8bit.v:12: error: invalid module item.

resultsalu_8bit.v:13: syntax error

resultsalu_8bit.v:13: error: invalid module item.

resultsalu_8bit.v:14: syntax error

resultsalu_8bit.v:14: error: invalid module item.

resultsalu_8bit.v:15: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 7.6s

⚙️  iverilog:

resultsalu_8bit.v:9: syntax error

resultsalu_8bit.v:9: error: Invalid module instantiation

resultsalu_8bit.v:12: error: invalid module item.

resultsalu_8bit.v:13: syntax error

resultsalu_8bit.v:13: error: invalid module item.

resultsalu_8bit.v:14: syntax error

resultsalu_8bit.v:14: error: invalid module item.

resultsalu_8bit.v:15: syntax error

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: dff_sync_reset  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 1.4s

⚙️  sim:

FAIL: expected q=0 got q=x (rst=1 d=1)

FAIL: expected q=0 got q=x (rst=1 d=1)

FAIL: expected q=0 got q=x (rst=1 d=1)

FAIL: expected q=1 got q=x (rst=0 d=1)

FAIL: expected q=0 got q=x (rst=0 d=0)

FAIL: expected q=1 got q=x (rst=0 d=1)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 2.7s

✅ PASSED on iteration 2  (4.2s total)

============================================================

AutoChip: counter_4bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.7s

✅ PASSED on iteration 1  (2.8s total)

============================================================

AutoChip: lfsr_8bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.1s

⚙️  iverilog:

resultslfsr_8bit.v:23: syntax error

resultslfsr_8bit_tb.v:7: error: invalid module item.

resultslfsr_8bit_tb.v:10: lfsr_out wire definition conflicts with reg definition at resultslfsr_8bit.v:7.

resultslfsr_8bit_tb.v:10: error: Net ``lfsr_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.4s

⚙️  iverilog:

resultslfsr_8bit.v:23: syntax error

resultslfsr_8bit_tb.v:7: error: invalid module item.

resultslfsr_8bit_tb.v:10: lfsr_out wire definition conflicts with reg definition at resultslfsr_8bit.v:7.

resultslfsr_8bit_tb.v:10: error: Net ``lfsr_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 7.4s

⚙️  iverilog:

resultslfsr_8bit.v:23: syntax error

resultslfsr_8bit_tb.v:7: error: invalid module item.

resultslfsr_8bit_tb.v:10: lfsr_out wire definition conflicts with reg definition at resultslfsr_8bit.v:7.

resultslfsr_8bit_tb.v:10: error: Net ``lfsr_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 7.5s

⚙️  iverilog:

resultslfsr_8bit.v:23: syntax error

resultslfsr_8bit_tb.v:7: error: invalid module item.

resultslfsr_8bit_tb.v:10: lfsr_out wire definition conflicts with reg definition at resultslfsr_8bit.v:7.

resultslfsr_8bit_tb.v:10: error: Net ``lfsr_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 7.6s

⚙️  iverilog:

resultslfsr_8bit.v:23: syntax error

resultslfsr_8bit_tb.v:7: error: invalid module item.

resultslfsr_8bit_tb.v:10: lfsr_out wire definition conflicts with reg definition at resultslfsr_8bit.v:7.

resultslfsr_8bit_tb.v:10: error: Net ``lfsr_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.4s

⚙️  iverilog:

resultspwm_generator_tb.v:1: error: `timescale directive can not be inside a module definition.

resultspwm_generator_tb.v:5: syntax error

resultspwm_generator_tb.v:5: error: missing endmodule or attempt to nest modules.

resultspwm_generator.v:1: error: original module (pwm_generator) defined here.

resultspwm_generator_tb.v:8: error: Net ``pwm_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 3.5s

⚙️  iverilog:

resultspwm_generator_tb.v:1: error: `timescale directive can not be inside a module definition.

resultspwm_generator_tb.v:5: syntax error

resultspwm_generator_tb.v:5: error: missing endmodule or attempt to nest modules.

resultspwm_generator.v:1: error: original module (pwm_generator) defined here.

resultspwm_generator_tb.v:8: error: Net ``pwm_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 3.9s

⚙️  iverilog:

resultspwm_generator_tb.v:1: error: `timescale directive can not be inside a module definition.

resultspwm_generator_tb.v:5: syntax error

resultspwm_generator_tb.v:5: error: missing endmodule or attempt to nest modules.

resultspwm_generator.v:3: error: original module (pwm_generator) defined here.

resultspwm_generator_tb.v:8: error: Net ``pwm_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 3.9s

⚙️  iverilog:

resultspwm_generator_tb.v:1: error: `timescale directive can not be inside a module definition.

resultspwm_generator_tb.v:5: syntax error

resultspwm_generator_tb.v:5: error: missing endmodule or attempt to nest modules.

resultspwm_generator.v:3: error: original module (pwm_generator) defined here.

resultspwm_generator_tb.v:8: error: Net ``pwm_out'' has already been declared.

❌ duplicate_module  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 4.1s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.5s

✅ PASSED on iteration 1  (3.6s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.6s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:10: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:12: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 5.8s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

resultssimple_cpu_alu.v:10: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:11: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:12: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

resultssimple_cpu_alu.v:13: error: Unable to bind wire/reg/memory `B' in `simple_cpu_alu_tb.uut'

5 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 6.3s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 6.0s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 6.0s

⚙️  iverilog:

resultssimple_cpu_alu_tb.v:9: error: port ``B'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: fsm_seq_detector  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.4s

✅ PASSED on iteration 1  (8.5s total)

============================================================

AutoChip: uart_tx  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.6s

⚙️  iverilog:

resultsuart_tx.v:36: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:54: error: Incomprehensible case expression.

resultsuart_tx.v:56: syntax error

resultsuart_tx_tb.v:13: error: malformed statement

resultsuart_tx_tb.v:13: error: Incomprehensible case expression.

resultsuart_tx_tb.v:14: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 17.9s

⚙️  iverilog:

resultsuart_tx.v:36: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:54: error: Incomprehensible case expression.

resultsuart_tx.v:56: syntax error

resultsuart_tx_tb.v:13: error: malformed statement

resultsuart_tx_tb.v:13: error: Incomprehensible case expression.

resultsuart_tx_tb.v:14: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 18.2s

⚙️  iverilog:

resultsuart_tx.v:36: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:54: error: Incomprehensible case expression.

resultsuart_tx.v:56: syntax error

resultsuart_tx_tb.v:13: error: malformed statement

resultsuart_tx_tb.v:13: error: Incomprehensible case expression.

resultsuart_tx_tb.v:14: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 18.5s

⚙️  iverilog:

resultsuart_tx.v:36: syntax error

resultsuart_tx.v:39: error: Incomprehensible case expression.

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:54: error: Incomprehensible case expression.

resultsuart_tx.v:56: syntax error

resultsuart_tx_tb.v:13: error: malformed statement

resultsuart_tx_tb.v:13: error: Incomprehensible case expression.

resultsuart_tx_tb.v:14: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 18.0s

⚙️  iverilog:

resultsuart_tx.v:33: syntax error

resultsuart_tx.v:36: error: Incomprehensible case expression.

resultsuart_tx.v:48: syntax error

resultsuart_tx.v:51: error: Incomprehensible case expression.

resultsuart_tx.v:53: syntax error

resultsuart_tx_tb.v:13: error: malformed statement

resultsuart_tx_tb.v:13: error: Incomprehensible case expression.

resultsuart_tx_tb.v:14: syntax error

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 9.0s

✅ PASSED on iteration 1  (9.1s total)

⚠️  Dependency alu_8bit.v not found for alu_accumulator_top (did the dependency module pass?)

============================================================

AutoChip: alu_accumulator_top  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 6.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:14: error: Unknown module type: alu_8bit

2 error(s) during elaboration.

*** These modules were missing:

alu_8bit referenced 1 times.

***

❌ missing_module  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 6.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:14: error: Unknown module type: alu_8bit

2 error(s) during elaboration.

*** These modules were missing:

alu_8bit referenced 1 times.

***

❌ missing_module  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 6.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:14: error: Unknown module type: alu_8bit

2 error(s) during elaboration.

*** These modules were missing:

alu_8bit referenced 1 times.

***

❌ missing_module  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 6.5s

⚙️  iverilog:

resultsalu_accumulator_top.v:14: error: Unknown module type: alu_8bit

2 error(s) during elaboration.

*** These modules were missing:

alu_8bit referenced 1 times.

***

❌ missing_module  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 6.7s

⚙️  iverilog:

resultsalu_accumulator_top.v:14: error: Unknown module type: alu_8bit

2 error(s) during elaboration.

*** These modules were missing:

alu_8bit referenced 1 times.

***

❌ missing_module  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: param_register_file  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.7s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 4.9s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``wr_addr'' is not a port of uut.

resultsparam_register_file.v:15: error: Unable to bind wire/reg/memory `wr_addr' in `param_register_file_tb.uut'

2 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 4.9s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 4.9s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 4.9s

⚙️  iverilog:

resultsparam_register_file_tb.v:13: error: port ``rd_addr'' is not a port of uut.

1 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pipeline_mult_4x4  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.4s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 4.8s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 5.4s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 5.8s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 5.0s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: llama3.1

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 23.0s

⚙️  iverilog:

resultsspi_master_8bit.v:2: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 25.8s

⚙️  iverilog:

resultsspi_master_8bit.v:2: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 25.1s

⚙️  iverilog:

resultsspi_master_8bit.v:2: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 27.3s

⚙️  iverilog:

resultsspi_master_8bit.v:2: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 27.6s

⚙️  iverilog:

resultsspi_master_8bit.v:2: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: llama3.1

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1     8.97    0    0

full_adder                   easy         ✓      1     2.58    0    0

ripple_carry_adder           easy         ✗      -        -    5    0

comparator_8bit              easy         ✓      1     2.89    0    0

bcd_to_7seg                  easy         ✓      1     8.05    0    0

priority_enc_8               easy         ✓      1     6.35    0    0

alu_8bit                     medium       ✗      -        -    5    0

dff_sync_reset               medium       ✓      2     4.21    0    1

counter_4bit                 medium       ✓      1      2.8    0    0

lfsr_8bit                    medium       ✗      -        -    5    0

pwm_generator                medium       ✗      -        -    4    1

gray_counter_4bit            medium       ✓      1     3.59    0    0

simple_cpu_alu               hard         ✗      -        -    5    0

fsm_seq_detector             hard         ✓      1     8.53    0    0

uart_tx                      hard         ✗      -        -    5    0

sync_fifo_8                  hard         ✓      1     9.11    0    0

alu_accumulator_top          hard         ✗      -        -    5    0

param_register_file          critic       ✗      -        -    5    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✗      -        -    5    0

Pass@1: 10/20 = 50%

Summary → results\llama31\summary.json

# QWEN

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model qwen2.5-coder:14b

🚀 Running benchmark | Model: qwen2.5-coder:14b

============================================================

AutoChip: half_adder  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 18.9s

✅ PASSED on iteration 1  (19.0s total)

============================================================

AutoChip: full_adder  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 10.6s

✅ PASSED on iteration 1  (10.7s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 38.6s

✅ PASSED on iteration 1  (38.7s total)

============================================================

AutoChip: comparator_8bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.5s

✅ PASSED on iteration 1  (11.6s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 45.9s

✅ PASSED on iteration 1  (45.9s total)

============================================================

AutoChip: priority_enc_8  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 29.3s

✅ PASSED on iteration 1  (29.4s total)

============================================================

AutoChip: alu_8bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 34.1s

✅ PASSED on iteration 1  (34.2s total)

============================================================

AutoChip: dff_sync_reset  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.0s

✅ PASSED on iteration 1  (11.2s total)

============================================================

AutoChip: counter_4bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.7s

✅ PASSED on iteration 1  (11.8s total)

============================================================

AutoChip: lfsr_8bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 42.0s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 43.4s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 43.7s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 46.8s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 49.1s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.0s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 15.3s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 15.3s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 15.8s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 15.8s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 15.3s

✅ PASSED on iteration 1  (15.4s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 26.1s

✅ PASSED on iteration 1  (26.2s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 32.4s

✅ PASSED on iteration 1  (32.5s total)

============================================================

AutoChip: uart_tx  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 93.4s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 98.4s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 104.0s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 107.1s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 110.2s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 39.3s

✅ PASSED on iteration 1  (39.4s total)

============================================================

AutoChip: alu_accumulator_top  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 31.8s

✅ PASSED on iteration 1  (31.9s total)

============================================================

AutoChip: param_register_file  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 21.7s

✅ PASSED on iteration 1  (21.8s total)

============================================================

AutoChip: pipeline_mult_4x4  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 17.9s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 18.2s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 18.7s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 18.6s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 18.8s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: qwen2.5-coder:14b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 89.8s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 98.2s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 102.5s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 107.6s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 112.4s

⚙️  sim:

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: qwen2.5-coder:14b

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1     19.0    0    0

full_adder                   easy         ✓      1    10.71    0    0

ripple_carry_adder           easy         ✓      1    38.66    0    0

comparator_8bit              easy         ✓      1     11.6    0    0

bcd_to_7seg                  easy         ✓      1    45.95    0    0

priority_enc_8               easy         ✓      1    29.35    0    0

alu_8bit                     medium       ✓      1    34.18    0    0

dff_sync_reset               medium       ✓      1    11.15    0    0

counter_4bit                 medium       ✓      1    11.79    0    0

lfsr_8bit                    medium       ✗      -        -    0    5

pwm_generator                medium       ✗      -        -    0    5

gray_counter_4bit            medium       ✓      1    15.42    0    0

simple_cpu_alu               hard         ✓      1    26.24    0    0

fsm_seq_detector             hard         ✓      1    32.52    0    0

uart_tx                      hard         ✗      -        -    0    5

sync_fifo_8                  hard         ✓      1    39.37    0    0

alu_accumulator_top          hard         ✓      1    31.93    0    0

param_register_file          critic       ✓      1     21.8    0    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✗      -        -    0    5

Pass@1: 15/20 = 75%

Summary → results\qwen25-coder_14b\summary.json

DEEPSEEK

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model deepseek-coder:6.7b

🚀 Running benchmark | Model: deepseek-coder:6.7b

============================================================

AutoChip: half_adder  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 10.9s

⚙️  iverilog:

resultshalf_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultshalf_adder_tb.v:7: error: port ``cout'' is not a port of uut.

2 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.0s

⚙️  iverilog:

resultshalf_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultshalf_adder_tb.v:7: error: port ``cout'' is not a port of uut.

resultshalf_adder.v:3: error: out['sd0] is not a valid l-value in half_adder_tb.uut.

resultshalf_adder.v:1:      : out['sd0] is declared here as wire.

resultshalf_adder.v:4: error: out['sd1] is not a valid l-value in half_adder_tb.uut.

resultshalf_adder.v:1:      : out['sd1] is declared here as wire.

resultshalf_adder.v:2: warning: @* found no sensitivities so it will never trigger.

4 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 16.9s

⚙️  iverilog:

resultshalf_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultshalf_adder_tb.v:7: error: port ``cout'' is not a port of uut.

2 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 8.5s

⚙️  iverilog:

resultshalf_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultshalf_adder_tb.v:7: error: port ``cout'' is not a port of uut.

resultshalf_adder.v:3: error: out['sd0] is not a valid l-value in half_adder_tb.uut.

resultshalf_adder.v:1:      : out['sd0] is declared here as wire.

resultshalf_adder.v:4: error: out['sd1] is not a valid l-value in half_adder_tb.uut.

resultshalf_adder.v:1:      : out['sd1] is declared here as wire.

resultshalf_adder.v:2: warning: @* found no sensitivities so it will never trigger.

4 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 10.3s

⚙️  iverilog:

resultshalf_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultshalf_adder_tb.v:7: error: port ``cout'' is not a port of uut.

2 error(s) during elaboration.

❌ port_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: full_adder  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.5s

⚙️  iverilog:

resultsfull_adder_tb.v:7: error: port ``sum'' is not a port of uut.

resultsfull_adder_tb.v:7: error: port ``cout'' is not a port of uut.

resultsfull_adder.v:2: error: reg out; cannot be driven by primitives or continuous assignment.

resultsfull_adder.v:3: error: reg out; cannot be driven by primitives or continuous assignment.

4 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.8s

✅ PASSED on iteration 2  (13.5s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 13.9s

✅ PASSED on iteration 1  (14.1s total)

============================================================

AutoChip: comparator_8bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 6.3s

⚙️  iverilog:

resultscomparator_8bit.v:3: error: reg gt; cannot be driven by primitives or continuous assignment.

resultscomparator_8bit.v:4: error: reg eq; cannot be driven by primitives or continuous assignment.

resultscomparator_8bit.v:5: error: reg lt; cannot be driven by primitives or continuous assignment.

3 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.9s

✅ PASSED on iteration 2  (14.4s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 26.0s

✅ PASSED on iteration 1  (26.1s total)

============================================================

AutoChip: priority_enc_8  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 12.6s

⚙️  iverilog:

resultspriority_enc_8.v:4: error: valid is not a valid l-value in priority_enc_8_tb.uut.

resultspriority_enc_8.v:1:      : valid is declared here as wire.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 19.6s

⚙️  iverilog:

resultspriority_enc_8_tb.v:9: error: port ``valid'' is not a port of uut.

resultspriority_enc_8.v:6: error: valid is not a valid l-value in priority_enc_8_tb.uut.

resultspriority_enc_8.v:3:      : valid is declared here as wire.

2 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 15.9s

⚙️  iverilog:

resultspriority_enc_8_tb.v:9: error: port ``valid'' is not a port of uut.

resultspriority_enc_8.v:6: error: valid is not a valid l-value in priority_enc_8_tb.uut.

resultspriority_enc_8.v:3:      : valid is declared here as wire.

2 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 17.1s

⚙️  iverilog:

resultspriority_enc_8_tb.v:9: error: port ``valid'' is not a port of uut.

resultspriority_enc_8.v:6: error: valid is not a valid l-value in priority_enc_8_tb.uut.

resultspriority_enc_8.v:3:      : valid is declared here as wire.

2 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 18.7s

⚙️  iverilog:

resultspriority_enc_8_tb.v:9: error: port ``valid'' is not a port of uut.

resultspriority_enc_8.v:6: error: valid is not a valid l-value in priority_enc_8_tb.uut.

resultspriority_enc_8.v:3:      : valid is declared here as wire.

2 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: alu_8bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 11.9s

✅ PASSED on iteration 1  (12.0s total)

============================================================

AutoChip: dff_sync_reset  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.9s

✅ PASSED on iteration 1  (6.0s total)

============================================================

AutoChip: counter_4bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.9s

⚙️  iverilog:

resultscounter_4bit.v:5: syntax error

resultscounter_4bit_tb.v:1: error: `timescale directive can not be inside a module definition.

resultscounter_4bit_tb.v:2: error: malformed statement

resultscounter_4bit_tb.v:3: syntax error

resultscounter_4bit_tb.v:3: Syntax in assignment statement l-value.

resultscounter_4bit_tb.v:4: syntax error

resultscounter_4bit_tb.v:4: error: malformed statement

resultscounter_4bit_tb.v:5: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 7.0s

✅ PASSED on iteration 2  (10.0s total)

============================================================

AutoChip: lfsr_8bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 17.8s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 24.2s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h87

FAIL [AB step1]: expected 8'hC9 got 8'h2d

FAIL [AB step2]: expected 8'hF8 got 8'h56

FAIL [hold]: value changed when enable=0 (got 8'hab)

FAIL [hold2]: value changed when enable=0 (got 8'hab)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 26.0s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h87

FAIL [AB step1]: expected 8'hC9 got 8'h2d

FAIL [AB step2]: expected 8'hF8 got 8'h56

FAIL [hold]: value changed when enable=0 (got 8'hab)

FAIL [hold2]: value changed when enable=0 (got 8'hab)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 28.2s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h87

FAIL [AB step1]: expected 8'hC9 got 8'h2d

FAIL [AB step2]: expected 8'hF8 got 8'h56

FAIL [hold]: value changed when enable=0 (got 8'hab)

FAIL [hold2]: value changed when enable=0 (got 8'hab)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 30.7s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'h87

FAIL [AB step1]: expected 8'hC9 got 8'h2d

FAIL [AB step2]: expected 8'hF8 got 8'h56

FAIL [hold]: value changed when enable=0 (got 8'hab)

FAIL [hold2]: value changed when enable=0 (got 8'hab)

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 9.2s

⚙️  iverilog:

resultspwm_generator.v:9: error: reg pwm_out; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 9.0s

⚙️  iverilog:

resultspwm_generator.v:9: error: reg pwm_out; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 9.5s

⚙️  iverilog:

resultspwm_generator.v:9: error: reg pwm_out; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 10.1s

⚙️  iverilog:

resultspwm_generator.v:9: error: reg pwm_out; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 10.6s

⚙️  iverilog:

resultspwm_generator.v:9: error: reg pwm_out; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.0s

✅ PASSED on iteration 1  (7.1s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 10.3s

⚙️  iverilog:

resultssimple_cpu_alu.v:9: syntax error

resultssimple_cpu_alu.v:11: Syntax in assignment statement l-value.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 11.4s

✅ PASSED on iteration 2  (21.9s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 17.5s

⚙️  iverilog:

resultsfsm_seq_detector.v:16: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:17: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:18: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:19: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 20.8s

⚙️  iverilog:

resultsfsm_seq_detector.v:5: error: reg detected; cannot be driven by primitives or continuous assignment.

1 error(s) during elaboration.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 23.5s

⚙️  sim:

FAIL: in=1  expected detected=1  got detected=0 (at time 73000)

FAIL: in=1  expected detected=0  got detected=1 (at time 93000)

FAIL: in=1  expected detected=1  got detected=0 (at time 103000)

FAIL: in=1  expected detected=0  got detected=1 (at time 153000)

FAIL: in=1  expected detected=0  got detected=1 (at time 173000)

FAIL: in=1  expected detected=1  got detected=0 (at time 243000)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 27.9s

⚙️  iverilog:

resultsfsm_seq_detector.v:5: error: reg detected; cannot be driven by primitives or continuous assignment.

resultsfsm_seq_detector.v:16: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:17: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:18: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:19: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

❌ reg_wire_mismatch  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 28.0s

⚙️  iverilog:

resultsfsm_seq_detector.v:5: error: reg detected; cannot be driven by primitives or continuous assignment.

resultsfsm_seq_detector.v:16: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:17: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:18: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

resultsfsm_seq_detector.v:3:      : next_state is declared here as wire.

resultsfsm_seq_detector.v:19: error: next_state is not a valid l-value in fsm_seq_detector_tb.uut.

❌ reg_wire_mismatch  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: uart_tx  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 40.8s

⚙️  iverilog:

resultsuart_tx.v:51: syntax error

resultsuart_tx.v:51: error: malformed statement

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 49.2s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [a5]: busy still high after done

FAIL [a5]: tx_out not idle-high after done, got 0

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 56.3s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h02  (bits wrong)

FAIL [a5]: stop bit should be 1, got 0

FAIL [a5]: busy still high after done

FAIL [a5]: tx_out not idle-high after done, got 0

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 57.2s

⚙️  iverilog:

resultsuart_tx.v:1: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 13.0s

⚙️  iverilog:

resultsuart_tx.v:3: warning: macro uart_tx undefined (and assumed null) at this point.

resultsuart_tx.v:1: syntax error

I give up.

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.2s

⚙️  iverilog:

resultssync_fifo_8_tb.v:1: error: `timescale directive can not be inside a module definition.

resultssync_fifo_8_tb.v:5: syntax error

resultssync_fifo_8_tb.v:5: error: malformed statement

resultssync_fifo_8_tb.v:6: syntax error

resultssync_fifo_8_tb.v:6: Syntax in assignment statement l-value.

resultssync_fifo_8_tb.v:7: syntax error

resultssync_fifo_8_tb.v:7: Syntax in assignment statement l-value.

resultssync_fifo_8_tb.v:8: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 20.7s

✅ PASSED on iteration 2  (37.1s total)

============================================================

AutoChip: alu_accumulator_top  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 17.0s

✅ PASSED on iteration 1  (17.1s total)

============================================================

AutoChip: param_register_file  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.1s

✅ PASSED on iteration 1  (8.2s total)

============================================================

AutoChip: pipeline_mult_4x4  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 17.5s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 14.8s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got x

FAIL [5*6]: expected product=30 at cycle 3, got x

FAIL [2*3]: expected 6 at cycle 5, got x

FAIL [7*7]: expected 49 at cycle 6, got x

FAIL [1*1]: expected 1 at cycle 7, got x

FAIL [15*15]: expected 225, got x

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 15.5s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got x

FAIL [5*6]: expected product=30 at cycle 3, got x

FAIL [2*3]: expected 6 at cycle 5, got x

FAIL [7*7]: expected 49 at cycle 6, got x

FAIL [1*1]: expected 1 at cycle 7, got x

FAIL [15*15]: expected 225, got x

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 16.7s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got x

FAIL [5*6]: expected product=30 at cycle 3, got x

FAIL [2*3]: expected 6 at cycle 5, got x

FAIL [7*7]: expected 49 at cycle 6, got x

FAIL [1*1]: expected 1 at cycle 7, got x

FAIL [15*15]: expected 225, got x

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 17.8s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got x

FAIL [5*6]: expected product=30 at cycle 3, got x

FAIL [2*3]: expected 6 at cycle 5, got x

FAIL [7*7]: expected 49 at cycle 6, got x

FAIL [1*1]: expected 1 at cycle 7, got x

FAIL [15*15]: expected 225, got x

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: deepseek-coder:6.7b

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 26.2s

⚙️  iverilog:

resultsspi_master_8bit_tb.v:1: error: `timescale directive can not be inside a module definition.

resultsspi_master_8bit_tb.v:9: syntax error

resultsspi_master_8bit_tb.v:9: error: malformed statement

resultsspi_master_8bit_tb.v:10: syntax error

resultsspi_master_8bit_tb.v:10: Syntax in assignment statement l-value.

resultsspi_master_8bit_tb.v:12: syntax error

resultsspi_master_8bit_tb.v:12: Syntax in assignment statement l-value.

resultsspi_master_8bit_tb.v:13: syntax error

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 46.9s

⚙️  sim:

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

FAIL [cs_n]: cs_n should be high after transfer done, got 0

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 49.9s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 51.4s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 49.9s

⚙️  sim:

FAIL [start]: cs_n should go low after start

FAIL: done never asserted within timeout

FAIL [miso]: expected miso_capture=8'hA5 got 8'hxx

FAIL [mosi]: expected mosi_captured=8'hB4 got 8'hxx

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: deepseek-coder:6.7b

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✗      -        -    5    0

full_adder                   easy         ✓      2    13.48    1    0

ripple_carry_adder           easy         ✓      1    14.05    0    0

comparator_8bit              easy         ✓      2    14.38    1    0

bcd_to_7seg                  easy         ✓      1    26.13    0    0

priority_enc_8               easy         ✗      -        -    5    0

alu_8bit                     medium       ✓      1    12.01    0    0

dff_sync_reset               medium       ✓      1     5.96    0    0

counter_4bit                 medium       ✓      2     10.0    1    0

lfsr_8bit                    medium       ✗      -        -    0    5

pwm_generator                medium       ✗      -        -    5    0

gray_counter_4bit            medium       ✓      1     7.13    0    0

simple_cpu_alu               hard         ✓      2    21.86    1    0

fsm_seq_detector             hard         ✗      -        -    4    1

uart_tx                      hard         ✗      -        -    3    2

sync_fifo_8                  hard         ✓      2    37.13    1    0

alu_accumulator_top          hard         ✓      1    17.11    0    0

param_register_file          critic       ✓      1     8.23    0    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✗      -        -    1    4

Pass@1: 12/20 = 60%

Summary → results\deepseek-coder_67b\summary.json

# GEMINI FLASH

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model gemini-2.5-flash

🚀 Running benchmark | Model: gemini-2.5-flash

============================================================

AutoChip: half_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.4s

✅ PASSED on iteration 1  (3.5s total)

============================================================

AutoChip: full_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.4s

✅ PASSED on iteration 1  (3.5s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.5s

✅ PASSED on iteration 1  (5.6s total)

============================================================

AutoChip: comparator_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.9s

✅ PASSED on iteration 1  (3.0s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.6s

✅ PASSED on iteration 1  (3.7s total)

============================================================

AutoChip: priority_enc_8  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.7s

✅ PASSED on iteration 1  (5.8s total)

============================================================

AutoChip: alu_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.5s

✅ PASSED on iteration 1  (4.6s total)

============================================================

AutoChip: dff_sync_reset  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.6s

✅ PASSED on iteration 1  (3.7s total)

============================================================

AutoChip: counter_4bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.1s

✅ PASSED on iteration 1  (3.2s total)

============================================================

AutoChip: lfsr_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 24.4s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.2s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 37.1s

⚙️  sim:

FAIL [duty=0]: pwm_out=1 at counter=255 (expected always 0)

FAIL [duty=4]: counter=255 expected low, got high

FAIL [duty=4]: counted 5 high cycles (expected 4)

FAIL [duty=255]: counted 256 high cycles (expected 255)

FAIL [duty=128]: counted 129 high cycles (expected 128)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 19.3s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: simple_cpu_alu  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.7s

✅ PASSED on iteration 1  (3.8s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.2s

✅ PASSED on iteration 1  (16.3s total)

============================================================

AutoChip: uart_tx  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 20.2s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h4a  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'h78  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 260.8s

Traceback (most recent call last):

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST\autochip_runner.py", line 786, in <module>

m = autochip_loop(config["spec"], mod_name, args.model, dep_files)

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST\autochip_runner.py", line 330, in autochip_loop

success, feedback = run_verification(iter_dir, module_name)

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST\autochip_runner.py", line 141, in run_verification

comp_result = subprocess.run(comp_cmd, capture_output=True, text=True, encoding="utf-8")

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv\Lib\subprocess.py", line 550, in run

stdout, stderr = process.communicate(input, timeout=timeout)

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv\Lib\subprocess.py", line 1209, in communicate

stdout, stderr = self._communicate(input, endtime, timeout)

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv\Lib\subprocess.py", line 1628, in _communicate

self.stdout_thread.join(self._remaining_time(endtime))

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv\Lib\threading.py", line 1149, in join

self._wait_for_tstate_lock()

File "C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv\Lib\threading.py", line 1169, in _wait_for_tstate_lock

if lock.acquire(block, timeout):

^^^^^^^^^^^^^^^^^^^^^^^^^^^^

KeyboardInterrupt

^C

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model gemini-2.5-flash

🚀 Running benchmark | Model: gemini-2.5-flash

============================================================

AutoChip: half_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.2s

✅ PASSED on iteration 1  (3.3s total)

============================================================

AutoChip: full_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.8s

✅ PASSED on iteration 1  (2.9s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.8s

✅ PASSED on iteration 1  (5.9s total)

============================================================

AutoChip: comparator_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.5s

✅ PASSED on iteration 1  (3.6s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.6s

✅ PASSED on iteration 1  (4.7s total)

============================================================

AutoChip: priority_enc_8  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.3s

✅ PASSED on iteration 1  (7.4s total)

============================================================

AutoChip: alu_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.6s

✅ PASSED on iteration 1  (4.7s total)

============================================================

AutoChip: dff_sync_reset  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.1s

✅ PASSED on iteration 1  (3.2s total)

============================================================

AutoChip: counter_4bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: lfsr_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.0s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 6.1s

⚙️  sim:

FAIL [duty=4]: counter=3 expected high, got low

FAIL [duty=4]: counter=255 expected low, got high

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.2s

✅ PASSED on iteration 1  (4.3s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.7s

✅ PASSED on iteration 1  (3.8s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: uart_tx  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: alu_accumulator_top  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: param_register_file  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: pipeline_mult_4x4  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: gemini-2.5-flash

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

=================================================================

SUMMARY — Model: gemini-2.5-flash

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1     3.32    0    0

full_adder                   easy         ✓      1     2.92    0    0

ripple_carry_adder           easy         ✓      1     5.88    0    0

comparator_8bit              easy         ✓      1     3.57    0    0

bcd_to_7seg                  easy         ✓      1      4.7    0    0

priority_enc_8               easy         ✓      1     7.39    0    0

alu_8bit                     medium       ✓      1     4.73    0    0

dff_sync_reset               medium       ✓      1      3.2    0    0

counter_4bit                 medium       ✗      -        -    0    0

lfsr_8bit                    medium       ✗      -        -    0    1

pwm_generator                medium       ✗      -        -    0    1

gray_counter_4bit            medium       ✓      1     4.27    0    0

simple_cpu_alu               hard         ✓      1     3.75    0    0

fsm_seq_detector             hard         ✗      -        -    0    0

uart_tx                      hard         ✗      -        -    0    0

sync_fifo_8                  hard         ✗      -        -    0    0

alu_accumulator_top          hard         ✗      -        -    0    0

param_register_file          critic       ✗      -        -    0    0

pipeline_mult_4x4            critic       ✗      -        -    0    0

spi_master_8bit              critic       ✗      -        -    0    0

Pass@1: 10/20 = 50%

Summary → results\gemini-25-flash\summary.json

# GEMINI PRO

(C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\venv) C:\KarDRIVE\BITS Files\3-2\LLM_Verilog Project\AgenticAI\AutoChipFinalTEST>python autochip_runner.py --model gemini-2.5-pro

🚀 Running benchmark | Model: gemini-2.5-pro

============================================================

AutoChip: half_adder  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.0s

✅ PASSED on iteration 1  (3.1s total)

============================================================

AutoChip: full_adder  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.9s

✅ PASSED on iteration 1  (3.0s total)

============================================================

AutoChip: ripple_carry_adder  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.6s

✅ PASSED on iteration 1  (7.7s total)

============================================================

AutoChip: comparator_8bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 2.1s

✅ PASSED on iteration 1  (2.2s total)

============================================================

AutoChip: bcd_to_7seg  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.6s

✅ PASSED on iteration 1  (3.7s total)

============================================================

AutoChip: priority_enc_8  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.8s

✅ PASSED on iteration 1  (5.9s total)

============================================================

AutoChip: alu_8bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.9s

✅ PASSED on iteration 1  (5.0s total)

============================================================

AutoChip: dff_sync_reset  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.1s

✅ PASSED on iteration 1  (3.2s total)

============================================================

AutoChip: counter_4bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.5s

✅ PASSED on iteration 1  (4.6s total)

============================================================

AutoChip: lfsr_8bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 9.5s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 60.2s

⚙️  sim:

FAIL [FF step1]: expected 8'hE3 got 8'hc7

FAIL [AB step1]: expected 8'hC9 got 8'hed

FAIL [AB step2]: expected 8'hF8 got 8'hce

FAIL [hold]: value changed when enable=0 (got 8'h67)

FAIL [hold2]: value changed when enable=0 (got 8'h67)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 70.5s

⚙️  sim:

FAIL [hold]: value changed when enable=0 (got 8'h7c)

FAIL [hold2]: value changed when enable=0 (got 8'h7c)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: pwm_generator  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: gray_counter_4bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 4.4s

✅ PASSED on iteration 1  (4.5s total)

============================================================

AutoChip: simple_cpu_alu  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 3.2s

✅ PASSED on iteration 1  (3.3s total)

============================================================

AutoChip: fsm_seq_detector  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 9.8s

✅ PASSED on iteration 1  (9.9s total)

============================================================

AutoChip: uart_tx  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 24.5s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h4a  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'h78  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 123.9s

⚙️  sim:

FAIL [a5]: busy not asserted after start

FAIL [a5]: start bit should be 0, got 1

FAIL [a5]: sent 8'ha5  captured 8'h4a  (bits wrong)

FAIL [3c]: busy not asserted after start

FAIL [3c]: start bit should be 0, got 1

FAIL [3c]: sent 8'h3c  captured 8'h78  (bits wrong)

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 264.4s

⚙️  iverilog:

resultsuart_tx.v:80: syntax error

resultsuart_tx.v:80: error: malformed statement

❌ syntax_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 29.0s

⚙️  sim:

FAIL [a5]: sent 8'ha5  captured 8'h4a  (bits wrong)

FAIL [3c]: sent 8'h3c  captured 8'h78  (bits wrong)

FAIL [3c]: stop bit should be 1, got 0

FAIL [ff]: sent 8'hff  captured 8'hfe  (bits wrong)

FAIL [00]: stop bit should be 1, got 0

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 115.1s

⚙️  iverilog:

resultsuart_tx.v:39: syntax error

resultsuart_tx.v:39: Syntax in assignment statement l-value.

resultsuart_tx.v:40: syntax error

resultsuart_tx.v:40: Syntax in assignment statement l-value.

resultsuart_tx.v:41: syntax error

resultsuart_tx.v:41: Syntax in assignment statement l-value.

resultsuart_tx.v:42: syntax error

resultsuart_tx.v:42: Syntax in assignment statement l-value.

❌ syntax_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: sync_fifo_8  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 7.5s

✅ PASSED on iteration 1  (7.6s total)

============================================================

AutoChip: alu_accumulator_top  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 5.6s

✅ PASSED on iteration 1  (5.7s total)

============================================================

AutoChip: param_register_file  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 8.9s

⚙️  iverilog:

resultsparam_register_file.v:8: error: Range expressions must be constant.

resultsparam_register_file.v:8       : This MSB expression violates the rule: ($clogb2(N))-('sd1)

resultsparam_register_file.v:7: error: Range expressions must be constant.

resultsparam_register_file.v:7       : This MSB expression violates the rule: ($clogb2(N))-('sd1)

resultsparam_register_file_tb.v:13: warning: Port 3 (wr_addr) of param_register_file expects 1 bits, got 2.

resultsparam_register_file_tb.v:13:        : Pruning 1 high bits of the expression.

resultsparam_register_file_tb.v:13: warning: Port 4 (rd_addr) of param_register_file expects 1 bits, got 2.

resultsparam_register_file_tb.v:13:        : Pruning 1 high bits of the expression.

❌ compile_other  — sending feedback to LLM

🔄 Iteration 2/5

❌ LLM Error: 503 UNAVAILABLE. {'error': {'code': 503, 'message': 'This model is currently experiencing high demand. Spikes in demand are usually temporary. Please try again later.', 'status': 'UNAVAILABLE'}}

❌ FAILED after 5 iterations

============================================================

AutoChip: pipeline_mult_4x4  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 16.4s

⚙️  sim:

FAIL [5*6]: expected product=30 at cycle 3, got 12

❌ logic_error  — sending feedback to LLM

🔄 Iteration 2/5

🤖 LLM responded in 71.2s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got 0

FAIL [5*6]: expected product=30 at cycle 3, got 12

FAIL [2*3]: expected 6 at cycle 5, got 30

FAIL [7*7]: expected 49 at cycle 6, got 6

FAIL [1*1]: expected 1 at cycle 7, got 49

FAIL [15*15]: expected 225, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 3/5

🤖 LLM responded in 251.7s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got 0

FAIL [5*6]: expected product=30 at cycle 3, got 12

FAIL [2*3]: expected 6 at cycle 5, got 30

FAIL [7*7]: expected 49 at cycle 6, got 6

FAIL [1*1]: expected 1 at cycle 7, got 49

FAIL [15*15]: expected 225, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 4/5

🤖 LLM responded in 88.3s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got 0

FAIL [5*6]: expected product=30 at cycle 3, got 12

FAIL [2*3]: expected 6 at cycle 5, got 30

FAIL [7*7]: expected 49 at cycle 6, got 6

FAIL [1*1]: expected 1 at cycle 7, got 49

FAIL [15*15]: expected 225, got 1

❌ logic_error  — sending feedback to LLM

🔄 Iteration 5/5

🤖 LLM responded in 116.4s

⚙️  sim:

FAIL [3*4]: expected product=12 after 2 cycles, got 0

FAIL [5*6]: expected product=30 at cycle 3, got 12

FAIL [2*3]: expected 6 at cycle 5, got 30

FAIL [7*7]: expected 49 at cycle 6, got 6

FAIL [1*1]: expected 1 at cycle 7, got 49

FAIL [15*15]: expected 225, got 1

❌ logic_error  — sending feedback to LLM

❌ FAILED after 5 iterations

============================================================

AutoChip: spi_master_8bit  |  Model: gemini-2.5-pro

============================================================

🔄 Iteration 1/5

🤖 LLM responded in 38.9s

✅ PASSED on iteration 1  (39.0s total)

=================================================================

SUMMARY — Model: gemini-2.5-pro

=================================================================

Module                       Lvl       Pass  Iters  Time(s)   CE   SE

---------------------------- -------- ----- ------ -------- ---- ----

half_adder                   easy         ✓      1     3.13    0    0

full_adder                   easy         ✓      1     3.03    0    0

ripple_carry_adder           easy         ✓      1      7.7    0    0

comparator_8bit              easy         ✓      1     2.23    0    0

bcd_to_7seg                  easy         ✓      1     3.68    0    0

priority_enc_8               easy         ✓      1     5.86    0    0

alu_8bit                     medium       ✓      1     5.02    0    0

dff_sync_reset               medium       ✓      1     3.21    0    0

counter_4bit                 medium       ✓      1     4.63    0    0

lfsr_8bit                    medium       ✗      -        -    0    3

pwm_generator                medium       ✗      -        -    0    0

gray_counter_4bit            medium       ✓      1     4.46    0    0

simple_cpu_alu               hard         ✓      1     3.31    0    0

fsm_seq_detector             hard         ✓      1     9.94    0    0

uart_tx                      hard         ✗      -        -    2    3

sync_fifo_8                  hard         ✓      1     7.64    0    0

alu_accumulator_top          hard         ✓      1     5.66    0    0

param_register_file          critic       ✗      -        -    1    0

pipeline_mult_4x4            critic       ✗      -        -    0    5

spi_master_8bit              critic       ✓      1    39.03    0    0

Pass@1: 15/20 = 75%

Summary → results\gemini-25-pro\summary.json